TABLE OF CONTENTS

PAGE

1.0	INTRODUCTION	3

2.0	PURPOSE	3

3.0	STANDARDS & SPECIFICATIONS	5

3.1	Overview	5

3.2	Horizontal Accuracy	6

3.3	Vertical Accuracy	6

4.0	QUALITY REVIEW DOCUMENTATION	6

5.0	SUMMARY	10

TABLE OF CONTENTS (cont)

PAGE

LIST OF TABLES

Table of Contents	i

Table 2.1 Studied Stream Reaches	3

Table 4.1 Vertical Accuracy Statistic Worksheet	8

LIST OF FIGURES

Figure 4.1 Error Frequency Distribution	9

Figure 4.2 Error Frequency Distribution - Urban	9

Figure 4.3 Error Frequency Distribution - Grass	10

Figure 4.4 Error Frequency Distribution - Woods	10

LIST OF APPENDICES

Appendix A: CD-A with LIDAR Grid Data

Appendix B: CD-B with LIDAR Grid Data

Appendix C: CD-C with LIDAR Raw Data

Appendix D: CD-D with LIDAR Raw Data

Appendix E: CD-E with LIDAR Raw Data; Checkpoints shape file; TINs for Checkpoints

Appendix F: CD-F with Mass Point Data

Appendix G: CD-G with Mass Point Data

Appendix H: CD-H with Mass Point Data; Breakline data; Tiling scheme; Report

Appendix I: EnerQuest Systems LLC Quality Control Report

Appendix J: Metadata

# INTRODUCTION

Wyoming County is nested in the mountains of southern West Virginia. The county has four main towns Bailysville, Mullens, Pineville and Oceana with Pineville being the county seat.

# PURPOSE

FEMA has tasked AMEC Earth & Environmental, Inc. (AMEC) to update the Flood Insurance Study (FIS) and Flood Insurance Rate Maps (FIRM) for Wyoming County, West Virginia in accordance with FEMA’s Guidelines and Specifications for Flood Hazard Mapping Partners (G&S), dated February 2002.  For the FIS restudy AMEC has developed topographic mapping using the services of EnerQuest Systems, LLC, and DTMs to produce 1”=500’, 4 foot contour interval topographic mapping.  EnerQuest Systems LLC utilized Global Positioning System (GPS) surveys, performed by AMEC, to establish horizontal and vertical control for the mapping effort.  The streams to be studied by AMEC are identified below in Table 2.1.

Table 2.1  Studied Stream Reaches

| Watershed | Stream | Study Area Description | Study Type | Study Length (miles) |
| --- | --- | --- | --- | --- |
| Guyandotte River | Barkers Creek | From the confluence with the Guyandotte river to 2.5 miles upstream of Milam Branch | Limited Detail | 10.0 |
|  | Gooney Otter Creek | From the confluence with the Barkers Creek to 1.1 miles upstream of Noseman Branch | Limited Detail | 3.5 |
|  | Huff Creek | From the county boundary to Brushy Fork | Limited Detail | 9.5 |
|  | Indian Creek | From the confluence with the Guyandotte river to Raccoon Branch | Limited Detail | 7.5 |
|  | Muzzle Creek | From the confluence with the Suke Creek to 0.8 miles upstream of Right Fork | Limited Detail | 1.5 |
|  | Clear Fork | From the end of the existing detail study limits near Cow Creek to 0.2 miles upstream of Wolfpen Branch | Limited Detail | 2.5 |
|  | Clear Fork | From the confluence with the Guyandotte River to the limits of the existing detail study near Oceana | Approximate | 6.9 |
|  | Laurel Fork | Limits of existing A zones | Limited Detail | 11.8 |
|  | Slab Fork | From the limit of the existing detail study to Measle Fork | Limited Detail | 5.0 |
|  | Milam Fork | From the confluence with Laurel Fork to 3.5 miles upstream | Hydrologic Validation | 3.5 |
|  | Slab Fork | From the confluence with the Guyandotte river to 2.2 miles upstream | Hydrologic Validation | 2.2 |
|  | Rockcastle Creek | From the confluence with Bearhole creek upstream 0.8 miles | Hydrologic Validation | 0.8 |
|  | Bearhole Fork | From the confluence with the Guyandotte river to 0.9 miles upstream | Hydrologic Validation | 0.9 |
|  | Indian Creek | From Raccoon Branch to 0.3 miles downstream of White Oak Branch | Hydrologic Validation | 5.4 |
|  | Guyandotte River | From Sams Branch to the county boundary | Hydrologic Validation | 35.7 |
| Guyandotte River | Little Huff Creek | Limits of existing approximate study | Approximate | 11.7 |
|  | Pad Fork | Limits of existing approximate study | Approximate | 0.7 |
|  | Suke Creek | Limits of existing approximate study | Approximate | 0.6 |
|  | Big Club Creek | Limits of existing approximate study | Approximate | 4.0 |
|  | Guyandotte River | Limits of existing approximate study | Approximate | 18.2 |
|  | Marsh Fork | Limits of existing approximate study | Approximate | 2.1 |
|  | Indian Creek | Limits of existing approximate study | Approximate | 0.9 |
|  | Pinnacle Creek | Limits of existing approximate study | Approximate | 16.4 |
|  | Rockcastle Creek | Limits of existing approximate study | Approximate | 4.2 |
|  | Bearhole Creek | Limits of existing approximate study | Approximate | 1.6 |
|  | Cabin Creek | Limits of existing approximate study | Approximate | 0.6 |
|  | Still Run | Limits of existing approximate study | Approximate | 1.8 |
|  | Marsh Fork | Limits of existing approximate study | Approximate | 1.6 |
|  | Slab Fork | Limits of existing approximate study | Approximate | 1.4 |
|  | Upper Big Branch | Limits of existing approximate study | Approximate | 2.3 |
|  | Clear Fork | Limits of existing approximate study | Approximate | 6.8 |
|  | Toney Fork | Limits of existing approximate study | Approximate | 4.1 |
|  | Dry Branch | Limits of existing approximate study | Approximate | 0.3 |
|  | Upper Rd Branch | Limits of existing approximate study | Approximate | 0.2 |
|  | Lynco Branch | Limits of existing approximate study | Approximate | 0.8 |
|  | McDowell Mill Creek | Limits of existing approximate study | Approximate | 1.4 |
|  | Reedy Branch | Limits of existing approximate study | Approximate | 1.0 |
|  | Doublecamp Branch | Limits of existing approximate study | Approximate | 0.7 |
|  | Long Branch | Limits of existing approximate study | Approximate | 1.0 |

FEMA tasked AMEC to develop topographic data to support engineering studies and mapping of flood risk in all of Wyoming County, West Virginia (Approximately 510 square mailes).  EnerQuest Systems, LLC used an airborne Light Detection and Ranging (LIDAR) survey system to collect the topographic data.

# STANDARDS & SPECIFICATIONS

## Overview

To produce the high-accuracy elevation data required for hydraulic analyses on the studied streams, ground surveys and LIDAR-generated mapping is used.  The LIDAR-derived data met the National Standard for Spatial Data Accuracy (NSSDA) that is required to produce topographic maps and products as outlined in the Guidelines and Specifications for Flood Hazard Mapping Partners (G&S), dated February 2002.  The guidelines state that the Root Mean Square Error (RMSE) cannot exceed 1.2 for 4-foot contour interval mapping and the NSSDA cannot exceed 2.4.

AMEC has used a statistical testing methodology, consistent with aforementioned G&S, for estimating the positional accuracy of points in digital geospatial data, with respect to georeferenced ground positions that provide higher accuracy.  Checkpoint surveys were performed to establish independent Quality Assurance/Quality Control (QA/QC) checks for more accurate ground points.  They were mostly used to check the vertical accuracy of LIDAR datasets.

Triangulated Irregular Networks (TINs) were developed from the LIDAR only to be used for QA/QC reviews.  The survey checkpoints were compared to the interpolated TIN values as part of the accuracy checks.

Keeping the mapping contractor unaware of the ground-surveyed checkpoints facilitated an independent third party review.  In addition, the land surveyors had no knowledge of the LIDAR datasets rendering the ground survey points completely impartial.  This process led to a completely independent third party review or double blind assessment of the LIDAR data.

As per the February 2002 G&S, TIN accuracy is evaluated for the main categories of ground cover in the study area.  These typical ground cover categories are surveyed for a minimum of 20 test points.  Increasing the number of test points increases the confidence level in the calculated root mean square error (RMSE) values and improves performance standards.

## Horizontal Accuracy

Please refer to the attached report Appendix F for the Quality Control report submitted by EnerQuest Systems LLC for checking horizontal accuracy.

## Vertical Accuracy

In accordance with the NSSDA recommendations the RMSE was computed for vertical accuracy.  The squared difference between the independent (ground-surveyed) elevations and test (LIDAR) elevations was averaged and the square root of that yielded the RMSE.  As per FEMA guidelines, vertical accuracy of the LIDAR dataset is 1.96 times the computed RMSE value.

# QUALITY REVIEW DOCUMENTATION

The LIDAR survey system, developed by EnerQuest Systems, LLC, gathered data necessary to create the digital elevation information.  The raw LIDAR data consisted of 651 tile polygons with respective grid, breakline, and mass point datasets.

The majority of artifacts have been removed and any remaining artifacts are mostly outside of the floodplain corridors.  The terrain of Wyoming County is quite steep and rocky, and due to the steep terrain the remaining artifacts appear to have minimal impact on the data.  The FEMA guidelines for checkpoint selection are critical in such circumstances.

A checkpoint survey was performed for independent QA/QC checks to determine vertical accuracy of LIDAR datasets.  Three major ground cover categories were identified and a minimum of 20 test points were surveyed/selected for each category.  Thus a minimum of 60 test points were selected for three major land cover types– urban, woods, and grass.  In accordance with FEMAs Guidelines and Specifications for Flood Hazard Mapping Partners, the survey checkpoints were compared to the interpolated LIDAR elevations.  The square root of the mean of the squared errors was computed as the vertical RMSE.  The computed RMSE for the entire dataset was computed to be 0.88 feet, within the NFIP standard of 1.2 feet.  Table 4.1 on the next page reflects the results of the accuracy analysis.

For more details on LIDAR data collection please refer to Appendix F for the Quality Control report submitted by EnerQuest Systems LLC.

Table 4.1  Vertical Accuracy Statistic Worksheet

Also an analysis of the error frequency distribution for all the checkpoints is shown in the following histogram plots.  Histograms were also used to analyze error frequency distributions in each ground cover categories.

Figure 4.1  Error Frequency Distribution

Figure 4.2  Error Frequency Distribution – Urban

Figure 4.3  Error Frequency Distribution – Woods

Figure 4.4  Error Frequency Distribution – Grass

# SUMMARY

Aerial photogrammetric LIDAR data was collected for all the reaches as per Task Order (002).  The LIDAR data was checked for errors on all streams as per the Guidelines and Specifications for Flood Hazard Mapping Partners, dated February 2002 and was found to be within acceptable limits.  The elevations derived from the TINs were checked against ground-surveyed points and found to be within acceptable ranges in accordance with the Guidelines and Specifications for Flood Hazard Mapping Partners, dated February 2002.

Appendix A:  LIDAR Backup Data

| Description: | Contains LIDAR Grid Data for Tiles 1 through 300 (zip files) |
| --- | --- |
| Comments: |  |

Appendix B:  LIDAR Backup Data

| Description: | Contains LIDAR Grid Data for Tiles 301 through 652 (zip files) |
| --- | --- |
| Comments: |  |

Appendix C:  LIDAR Backup Data

| Description: | Contains LIDAR Raw Data |
| --- | --- |
| Comments: |  |

Appendix D:  LIDAR Backup Data

| Description: | Contains LIDAR Raw Data |
| --- | --- |
| Comments: |  |

Appendix E:  LIDAR Backup Data

| Description: | Contains LIDAR Raw Data; Checkpoints shape file; TINs for Checkpoints |
| --- | --- |
| Comments: |  |

Appendix F:  LIDAR Backup Data

| Description: | Contains LIDAR Mass Point Data |
| --- | --- |
| Comments: |  |

Appendix G:  LIDAR Backup Data

| Description: | Contains LIDAR Mass Point Data |
| --- | --- |
| Comments: |  |

Appendix H:  LIDAR Backup Data

| Description: | Contains LIDAR Mass Point data; Breaklines (.DGNs); Tiling Scheme; Also contains electronic copy of the LIDAR report. |
| --- | --- |
| Comments: |  |

Appendix I:  EnerQuest Systems LLC Quality Control Report

Appendix J:  Meta Data