Hydraulic Analysis Technical Support Data Notebook

Task Order HSFE03-14-J-0016 for Upper Monongahela Watershed (HUC-8 05020003)

Marion County and Monongalia County, WV

FEMA Contract HSFEHQ-09-D-0369

May 2016

Federal Emergency Management Agency

Region III

Department of Homeland Security

615 Chestnut Street

One Independence Mall, Sixth Floor

Philadelphia, PA 19106

TECHNICAL SUPPORT DATA NOTEBOOK (TSDN)

for

Upper Monongahela Watershed HUC-8 05020003

Marion and Monongalia Counties, West Virginia

HYDRAULIC ANALYSIS TSDN

SUBMITTED BY:

RAMPP

8401 Arlington Blvd.

Fairfax, VA 22031

DATE SUBMITTED: May 2016

TABLE OF CONTENTS

Acronyms	iii

1.0	Task Summary	1

1.1	Introduction	1

1.2	Project Work Scope	3

1.3	Background	4

1.3.1	Historic Flooding	4

1.3.2	Existing Hydraulic Studies	5

2.0	Methodology	5

2.1	Approach	5

2.2	Model and Computer Tools Used	6

2.3	Topographic Data	6

2.4	Cross Section Generation	6

2.5	Streamline Generation	7

2.6	Parameter Estimation	7

2.6.1	Manning’s “n” Values	7

2.6.2	Expansion and Contraction Coefficients	7

2.7	Structures	8

2.8	Discharges	8

2.9	Modeling Considerations	9

2.9.1	Boundary Conditions	9

2.9.2	Channel Banks and Flowpaths	9

2.9.3	Ineffective Flow Areas	9

2.9.4	Blocked Obstructions	9

2.9.5	Floodway Modeling	10

2.9.6	Model Calibration	10

3.0	Analysis and Results	10

3.1	Water Surface Elevations	10

3.2	High Water Marks / USGS Rating Curves	10

3.3	Comparison to Effective	11

3.4	Discussion of Significant Differences in WSELs (>5 feet)	15

3.5	Special Considerations	17

4.0	Special Problems	21

5.0	Exceptions	21

6.0	Conclusions	21

7.0	References	22

Tables

Table 1: Summary of Hydraulic Methodologies	1

Table 2: Effective and New Study Types	5

Table 3: Manning’s Roughness Coefficients (Detailed Study Streams)	7

Table 4: Expansion and Contraction Coefficients	8

Table 5: Boundary Conditions Table	9

Table 6: USGS Gages for Model Calibration	10

Table 7: Comparison of Effective and New WSELs (NAVD88)	12

Table 8: Comparison of Floodway Widths	14

Table 9: Effective vs. Restudy Peak Discharge Comparison	15

Figures

Figure 1: Study Area Map	2

Figure 2: Observed vs. Computed Rating Curve for Deckers Creek at Gage 03062500	11

Figure 3: Hickman Run Culvert from Cross Sections 5353 to 8116	18

Figure 4: Hickman Run Culvert under State Route 310	18

Figure 5: Floodplains in the Vicinity of Mountaineer Field	19

Figure 6: Floodplains in the Vicinity of Monongahela Boulevard	20

Figure 7: Floodplains in the Vicinity of University Boulevard and Takoma Street	20

Appendices

Appendix A	TSDN Documents

- Deliverables Checklist

- Mapping Information Index

Appendix B	Hydraulic Analysis Supporting Data

Appendix C	Quality Assurance / Quality Control

Appendix D	Correspondence

Appendix E	CD/DVD with All Applicable Data

ACRONYMS

cfs 	cubic feet per second

DEM		Digital Elevation Model

FEMA		Federal Emergency Management Agency

FIRM		Flood Insurance Rate Map

FIS 		Flood Insurance Study

G&S		Guidelines and Specifications for Flood Hazard Mapping Partners, as amended

GIS 		Geographic Information System

HEC-HMS	Hydrologic Engineering Center – Hydrologic Modeling System

HEC-RAS	Hydrologic Engineering Center – River Analysis System

HUC		Hydrologic Unit Code

HWM		High Water Mark

LiDAR	Light Detection and Ranging

LOMR		Letter of Map Revision

MIP 		Mapping Information Platform

NAVD88	North American Vertical Datum of 1988

NFIP		National Flood Insurance Program

NGVD 29	National Geodetic Vertical Datum of 1929

PAMAP	Pennsylvania Digital Base Mapping Program

QA/QC	Quality Assurance / Quality Control

RAMPP	Risk Assessment, Mapping, and Planning Partners

TSDN 		Technical Support Data Notebook

USACE	U.S. Army Corps of Engineers

USGS 		U.S. Geological Survey

UTM		Universal Transverse Mercator

WSEL		Water Surface Elevation

WVDEP	West Virginia Department of Environmental Protection

# Task Summary

## Introduction

This hydraulic study was conducted for the Department of Homeland Security’s Federal Emergency Management Agency (FEMA) as part of updating the Flood Insurance Studies (FISs) and Flood Insurance Rate Maps (FIRMs) for Marion and Monongalia Counties in the Upper Monongahela Watershed. The study was performed by Risk Analysis, Mapping, and Planning Partners (RAMPP), under the Production and Technical Services task order HSFE03-14-J-0016. This analysis was performed in accordance with FEMA’s Guidelines and Specifications for Flood Hazard Mapping Partners, as amended (G&S).

The Upper Monongahela Watershed (Hydrologic Unit Code [HUC]-8 05020003) covers 7,340 square miles in southwest Pennsylvania and northern West Virginia. The majority of the watershed is located in West Virginia in Marion, Monongalia, Preston, Taylor, and Wetzel Counties. The remaining portion of the watershed is located in Fayette and Green Counties in western Pennsylvania. The horizontal projection used for this study was Universal Transverse Mercator (UTM) Zone 17N in meters. The vertical datum used for this study was North American Vertical Datum of 1988 (NAVD88).

Hydraulic computations and analyses for Upper Monongahela Watershed consisted of determining water surface elevations (WSELs) for the 10-, 4-, 2-, 1-, 1+-, and 0.2- percent-annual-chance floods, as well as a floodway analysis, for 15.2 miles of detailed study reaches. A summary of the study streams and hydraulic methodologies is provided in Table 1, and a map showing the study stream reaches is shown in Figure 1.

Table 1: Summary of Hydraulic Methodologies

| Flooding Source | Upstream Study Limit | Downstream Study Limit | Study Mileage | Hydraulic Study Methodology |
| --- | --- | --- | --- | --- |
| Burroughs Run | At Van Voorhis Road | Woodland Drive | 1.28 | HEC-RAS |
| Deckers Creek | Intersection of Tyrone Road and Earl L Core Road | Inlet to Monongahela River | 6.19 | HEC-RAS |
| Dents Run | North East of I-19 and I-79 intersection | Inlet to Monongahela River | 1.57 | HEC-RAS |
| Hickman Run | Intersection of E Gralton Road and Connecticut Ave | Inlet to Monongahela River | 1.94 | HEC-RAS |
| Pompano Run | Fountain View Drive | Laurel Street | 3.29 | HEC-RAS |
| Reuben Run | 0.24 miles downstream from CR-3312 circle | Inlet to Prickett Creek | 0.53 | HEC-RAS |
| Scratchers Run | South of Brookside Drive | Intersection of Prickett Creek and Scratchers Run | 0.38 | HEC-RAS |

HEC-RAS = Hydrologic Engineering Center – River Analysis System

Figure 1: Study Area Map

An explanation has been included in cases where these data do not meet the required standards and specifications. Additionally, the FIRM data and the related metadata file have been uploaded to the Mapping Information Platform (MIP). In addition, RAMPP will provide written certification that the hydraulic analyses meet minimum FEMA standards and specifications, along with certification of other sub-tasks, with the Preliminary Technical Support Data Notebook (TSDN), later in the study process.

## Project Work Scope

Scope: RAMPP performed hydraulic analyses for approximately 15.2 miles of Effective Zone AE. The modeling included the 10-, 4-, 2-, 1-, 1+-, and 2-percent-annual-chance events based on peak discharges computed under the hydrologic analyses. The hydraulic methods used for this analysis included field surveys, floodway analysis, and enhanced level hydraulic modeling of the 10-, 4-, 2-, 1-, 1+-, and 2-percent-annual-chance events.

RAMPP used the cross section and field data collected during the Field Survey task and the topographic data collected during the Terrain Development task, when appropriate, to perform the hydraulic analyses. The hydraulic analyses will be used to establish flood elevations and regulatory floodways for the subject flooding sources.

It should be noted that the original hydraulic study length was 23.2 miles, but this was reduced to 15.2 miles because of the large number of structures along Pompano Run and Burroughs Run. The new field survey data were collected for 31 structures and 8 natural cross section surveys along portions of Deckers Creek, Dents Run, Hickman Run, Pompano Run, Reuben Run, and Scratchers Run where new structures were added or that had significant updates after effective study. For existing structures, the effective models or structure inventories and Geographic Information System (GIS) terrain data were used for creating model geometries.

RAMPP used the FEMA CHECK-RAS program to verify the hydraulic analyses. To facilitate the independent Quality Assurance / Quality Control (QA/QC) review, RAMPP will provide explanations for unresolved messages from the CHECK-RAS program, where appropriate. In addition, RAMPP will address all concerns or questions raised during the independent QA/QC review regarding the hydraulic analyses.

RAMPP documented automated data processing and modeling algorithms for GIS-based modeling and provided the data to FEMA for review to ensure they were consistent with the standards outlined above. Digital datasets were documented and provided to FEMA for approval before performing the hydraulic analyses to ensure the datasets met minimum requirements. If non-commercial (i.e., custom developed) software was used for the analyses, then RAMPP provided full user documentation, technical algorithm documentation, and software to FEMA for review before performing the hydraulic analyses.

Standards: All hydraulic data work was performed in accordance with the FEMA standards. RAMPP performed internal QA/QC reviews for all hydraulics, and a separate RAMPP entity performed external QA/QC.

Deliverables: In accordance with FEMA’s G&S, RAMPP has made the following products available to FEMA by uploading the digital data to the MIP. A metadata file that complies with the National Flood Insurance Program (NFIP) Metadata Profiles Specifications also accompanies the uploaded compliant digital data. Additionally, the TSDN format described in FEMA’s G&S has been uploaded.

Digital profiles of the 10-, 4-, 2-, 1-, 1+-, and 0.2-percent-annual-chance events, representing existing conditions using the FEMA RASPLOT program or similar software

Metadata file

Digital Floodway Data Tables for each flooding source, exportable via RASPLOT 3.0

Digital hydraulic modeling (input and output) files

Digital tables with range of Manning’s “n” values

Explanations for unresolved messages from the CHECK-RAS program, as appropriate

Digital versions of all backup data used in the analyses

Digital versions of draft text for inclusion in the FIS report in editable format (.doc or .docx)

Deliverables include all input and output data, GIS data layers, and final

TSDN Report

A summary that describes the methodologies used, input and outputs, and provides the results of all automated or manual QA/QC review steps taken during the preparation of the FIRM as outlined in the approved QA/QC Plan

## Background

### Historic Flooding

The streams in the Upper Monongahela Watershed can flood during any season, although historically the main flood months are usually December through April. The most common causes of flooding are three types of storms: afternoon thunderstorms in the summer, frontal systems in winter or early spring, and tropical cyclones in late summer or early fall. Although most severe floods have been attributed to rainfall, floods occurring in early spring have been compounded by snowmelt.

The mountainous topography in the Western-Plateaus Region is conducive to rapid rising of streams, and fast runoff best described as flash flooding. On larger streams during late summer or fall, large frontal storms or decaying tropical storms yield the worst flooding, while on smaller streams, high-intensity thunderstorms may produce extreme flooding in smaller drainage areas. Strong thunderstorms may be the most dangerous because they can produce flash floods with little or no warning.

The greatest flood known to occur in the watershed was the historic flood of July 1888. Although the exact height is not known, the WSEL at Morgantown Locks and Dam is estimated to have risen to an elevation of 824 feet. Major floods occurred from Hurricanes Hazel in October 1954, Agnes in June 1972, and Juan in November 1985, causing 47 deaths and $500 million in damages. Other major flooding events have occurred in 1961, 1967, 1980, and the most recent in 2000 (FEMA, 2010).

### Existing Hydraulic Studies

Deckers Creek, Dents Run, Hickman Run, and the downstream reach of Pompano Run were previously studied by detailed methods. Burroughs Run, Reuben Run, Scratchers Run, and the upstream reach of Pompano Run were previously studied by approximate methods. The previous hydraulic studies for the Upper Monongahela Watershed study area are documented in the Effective FIS reports for Marion County, WV (FEMA, 2012), and Monongalia County, WV (FEMA, 2010). Table 2 lists the effective and new studies conducted in the watershed.

Table 2: Effective and New Study Types

| Study Stream Name | Effective Special Flood Hazard Area | New Study Type |
| --- | --- | --- |
| Burroughs Run | Zone A | Zone AE with Floodway |
| Deckers Creek | Zone AE with Floodway | Zone AE with Floodway |
| Dents Run | Zone AE with Floodway | Zone AE with Floodway |
| Hickman Run | Zone AE with Floodway | Zone AE with Floodway |
| Pompano Run | Downstream: Zone AE with Floodway; Upstream: Zone A | Zone AE with Floodway |
| Reuben Run | Zone A | Zone AE with Floodway |
| Scratchers Run | Zone A | Zone AE with Floodway |

In 2005, the City of Morgantown conducted a drainage study for Burroughs Run and Pompano Run. The goal of the study was evaluate alternatives to mitigate flooding and improve water quality on the two streams (Morgantown Utility Board, 2005). As a follow up to the drainage study in 2007, the City of Morgantown performed channel improvements on the portions of Burroughs Run. The Hydrologic Engineering Center – River Analysis System (HEC-RAS) model for Burroughs Run, which was provided by the city, incorporated the 2007 channel improvements. The city model was obtained and updated as a part of this study.  The hydraulic model for Pompano Run was not available. The floodplain boundaries and WSELs from the drainage study were not available for comparison.

# Methodology

## Approach

The analyses consisted of determining WSELs for the 10-, 4-, 2-, 1-, 1+-, and 0.2-percent-annual-chance flood events and floodways for the study streams listed in Table 1. The hydraulic analyses for this study used a steady-state riverine analysis. For the hydraulic simulations, all structures were assumed to remain fully functional and have unobstructed flows.

The flow rates used for each study reach were calculated as described in the Hydrologic Analysis task for Upper Monongahela Watershed, prepared and submitted previously by RAMPP (FEMA, 2015B).

## Model and Computer Tools Used

WSELs were determined using detailed methods; RAMPP used HEC-RAS version 4.1.0 and the HEC-GeoRAS ArcGIS extension version 10.0 to assist in the hydraulic computations. Cross sections were cut from the topographic data using GeoRAS in an Esri ArcMap version 10.0 GIS platform. RAMPP created HEC-RAS models for each individual stream.

The flood profiles were generated using FEMA’s RASPLOT software program. The draft profiles have been uploaded to the MIP as part of this hydraulics submittal.

## Topographic Data

A terrain model was created in the Topographic Data Development task (FEMA, 2015C) from available Light Detection and Ranging (LiDAR) data in the watershed. The LiDAR was flown by Woolpert from 2006 to 2008 in support of the Pennsylvania Digital Base Mapping Program (PAMAP) and was used for the topographic data development for the Pennsylvania portion of the Upper Monongahela Watershed. The topographic data for the West Virginia project area came from several different sources. The first source was LiDAR flown by West Virginia University’s Natural Resources Analysis Center for the West Virginia Department of Environmental Protection (WVDEP). In areas of Monongalia County where there was no coverage from LiDAR acquisitions, 2-foot contours and spot elevations were obtained from the WVDEP. For Marion County, LiDAR flown by CVI in 2009 was used. For the portion of the watershed located in Taylor County, the best available topographic data was U.S. Geological Survey (USGS) 1/9 arc-second resolution digital elevation model (DEM).

Two areas for terrain development were identified for detailed floodplain mapping in Marion County and Monongalia County. For Marion County, the topographic data used was the 2009 Marion County LiDAR and breaklines. For Monongalia County, the topographic data used were the WVDEP LiDAR and the 2010 Monongalia County 2-foot contours.

Contours were created from the terrain model for use in placing hydraulic cross sections, and the terrain model was used to build the cross section geometry and to map the raw floodplains.  In some instances, the terrain model did not pick up the full depth or shape of the channel or true height of the channel banks as noted in the field survey.

For example, the terrain model may not pick up data for steep slopes or steep banks. For cases where there was a difference between the terrain model and the contour data, engineering judgment was used to determine the floodplain boundary based on the field survey and contour data.  All topographic data for this study were referenced to the horizontal projection UTM Zone 17N in meters and to the vertical datum NAVD88.

## Cross Section Generation

The placement, spacing, and alignment of the cross sections were determined in accordance with FEMA’s G&S (FEMA, 2003) and the HEC-RAS Hydraulic Reference Manual by USACE (USACE, 2010). Hydraulic cross sections were placed approximately 500 feet apart. Additional cross sections were placed to account for significant profile inflection points (profile breaks), which is critical for accuracy in the development of flood elevations. Cross sections were extended to contain all flood frequencies.

## Streamline Generation

The streamlines used in the HEC-RAS models were digitized using the aerial imagery and 2-foot contours generated from the terrain data.

## Parameter Estimation

### Manning’s “n” Values

For the detailed study streams, each cross section was analyzed to ensure that the correct “n” value was assigned to each area of the cross section. This was done based on USGS land use data, topographic data, aerial imagery, field reconnaissance data, and available land use data. Engineering judgment was used in updating the “n” values to align with the field reconnaissance data. The Manning’s “n” values used in the HEC-RAS model are summarized in Table 3 (based on Chow, 1959).

Table 3: Manning’s Roughness Coefficients (Detailed Study Streams)

| Study Stream Name | Channel “n” Value | Overbank “n” Value |
| --- | --- | --- |
| Burroughs Run | 0.03 – 0.045 | 0.03 – 0.12 |
| Deckers Creek | 0.038 – 0.049 | 0.05-0.13 |
| Dents Run | 0.035 – 0.05 | 0.03 – 0.12 |
| Hickman Run | 0.040 – 0.05 | 0.03 – 0.13 |
| Pompano Run | 0.028 – 0.055 | 0.06 – 0.13 |
| Reuben Run | 0.05 – 0.055 | 0.03 – 0.12 |
| Scratchers Run | 0.04 – 0.05 | 0.05 – 0.12 |

### Expansion and Contraction Coefficients

Expansion and contraction loss coefficients were applied to all crossing structures in the HEC-RAS models to account for the additional energy losses, and also applied between cross sections to account for losses due to the varying width of the channel. The contraction and expansion coefficients used in hydraulic modeling are based on the HEC-RAS Hydraulic Reference Manual (USACE, 2010) and summarized in Table 4 below.

Table 4: Expansion and Contraction Coefficients

| Cross Section Type | Contraction Loss Coefficient | Expansion Loss Coefficient |
| --- | --- | --- |
| Natural Cross Sections | 0.1 | 0.3 |
| Bridges, Culverts, Inline Weirs, and Dams | 0.3 | 0.5 |

## Structures

Field survey was conducted by ESP Associates, P.A. for a total of 31 structures and 8 natural cross sections within the Upper Monongahela Watershed project area. The structures surveyed are listed in Appendix B. The details of the field survey are documented in the Field Survey TSDN, prepared by RAMPP (FEMA, 2015A).

To further support the hydraulic analysis, RAMPP conducted field reconnaissance to evaluate and document hydraulic characteristics of structures for which ESP Associates, P.A. did not survey. Measurements were estimate for the channel depth, deck height, rail height, and structure opening width or culvert size and shape, as applicable. During the field reconnaissance, RAMPP also obtained information regarding the channel and overbank conditions for determination of the Manning’s “n” values. The photos and approximate measurements were used to code the structures in the HEC-RAS models in conjunction with the Effective HEC-2 data obtained previously, if available.

During field reconnaissance, several structures could not be accessed according to safety protocols or without trespassing on private property. In cases where data do not meet the required standards and specifications, an explanation is included. A list of structures visited during field reconnaissance is available in Appendix B. The photos and measurements taken during field reconnaissance are in the supplemental data folder of this hydraulics submittal package.

The Effective HEC-2 data were obtained from FEMA as PDFs for Deckers Creek, Dents Run, and Hickman Run. The structure cross sections from the Effective HEC-2 data were used to determine the elevations for the top of road, low chord, opening width, and channel invert at the structure. The Effective HEC-2 structure data were compared to the field reconnaissance measurements, and where significantly different, the structure was modeled according to the field reconnaissance measurements with the understanding that structure and stream geometries may have altered since the Effective studies were performed.

## Discharges

Discharges for the study streams were developed using various methods. The hydrologic analysis for Deckers Creek, Dents Run, and Hickman Run, Reuben Run, and Scratchers Run was regression based. The hydrologic analysis for Burroughs Run and Pompano Run was HEC- Hydrologic Modeling System (HMS) based. The methods used to calculate discharges are presented in detail in the Hydrologic Analysis TSDN, prepared by RAMPP (FEMA, 2015B).

## Modeling Considerations

### Boundary Conditions

The downstream boundary conditions determined for each stream studied by detailed methods are shown in Table 5. The flow is assumed to be uniform, so the energy slope was estimated by measuring the channel bed slope. For study reaches that tie into Effective or restudy of the same stream, the known water surface elevations from those detailed studies were used as downstream boundary conditions for the hydraulic models.

Table 5: Boundary Conditions Table

| Study Stream Name | Boundary Condition |
| --- | --- |
| Burroughs Run | Normal Depth |
| Deckers Creek | Normal Depth |
| Dents Run | Normal Depth |
| Hickman Run | Normal Depth |
| Pompano Run | Normal Depth |
| Reuben Run | Normal Depth |
| Scratchers Run | Normal Depth |

### Channel Banks and Flowpaths

Channel banks and flowpath centerlines were initially created in HEC-GeoRAS, with the aerial imagery, contour data, and FEMA Effective floodplains being used to estimate the bank and flowpath locations. Once the geometry was imported into HEC-RAS, the channel banks were edited in each cross section so that they correctly represented the bank location, were at similar elevations on each side of a channel, and were below all of the flood frequency elevations in most cases.

### Ineffective Flow Areas

Ineffective flow areas were defined in the HEC-RAS cross sections where water in the cross section did not contribute to the flood conveyance. At structures, ineffective flow areas were applied based on a contraction ratio of 1:1 for the upstream side and an expansion ratio of 1:1 to 2:1 for the downstream side. Ineffective areas were considered for the profiles that did not overtop the roadway and were applied consistently on the upstream and downstream sides of the structure. Ineffective areas were also applied after reviewing the shape of the floodplain for cross sections with divided flows.

### Blocked Obstructions

Blocked obstructions were used in the HEC-RAS model for large buildings (approximately 100 feet or more in length) or structures otherwise deemed significant in terms of effect on flow conveyance. The blocked obstructions were manually delineated using aerial imagery. For densely populated areas, blocked obstructions were not modeled, but the Manning’s “n” values were increased to account for the numerous buildings in the floodplain.

### Floodway Modeling

Floodway analysis was performed for all study stream reaches based on equal-conveyance reduction from each side of the floodplain. Floodway widths were computed and mapped at cross sections. Between cross sections, the floodway boundaries were interpolated. NFIP guidelines mandate that surcharge values must be between 0.0 and 1.0 foot.

### Model Calibration

No high water marks (HWMs) were available in the study area for model calibration.

At the Deckers Creek USGS gage station (shown in Table 6), HEC-RAS models were calibrated using the observed rating curve at USGS gage Deckers Creek at Morgantown, WV. To calibrate to the observed values, the rating curve for gage data was obtained from the USGS website. The gage datum was then added to the gage heights to obtain the observed WSELs. The model was calibrated within 0.5 foot of the observed WSELs by raising the n value by up to 0.01.

Table 6: USGS Gages for Model Calibration

| USGS Gage Number | Flooding Source | Gage Location |
| --- | --- | --- |
| 03062500 | Deckers Creek | Deckers Creek at Morgantown, WV |

# Analysis and Results

RAMPP used the computed water surface elevations to develop the floodplain spatial file. This floodplain spatial file that was generated using the results of the hydraulic analysis is included with the digital data. The floodplain data in the S_FLD_HAZ_AR spatial file are raw outputs and will be edited during the Floodplain Mapping task. The only editing done to the raw floodplain data at this time has been to merge overlapping floodplains and remove isolated floodplains from the files.

## Water Surface Elevations

The calculated WSELs for the 10-, 4-, 2-, 1-, 1+-, and 0.2-percent-annual-chance flood events for detailed study streams are in the S_XS layer and L_XS_Elevation table.

## High Water Marks / USGS Rating Curves

No HWMs were available for the calibration of the HEC-RAS models.

Figure 2 shows the comparison between observed and computed rating curves at USGS gage 03062500 along Deckers Creek. The calibration within 0.5 foot of the rating curve WSELs was obtained by slightly revising the main channel Manning’s “n” values. The main channel Manning’s “n” values were raised by a value from 0.005 up to 0.01 to achieve modeled WSELs within 0.5 foot of the observed elevations.

Figure 2: Observed vs. Computed Rating Curve for Deckers Creek at Gage 03062500

## Comparison to Effective

The team compared the restudy results to the Effective studies for the WSELs, where available, and for floodplain boundary extents. For streams where a WSEL comparison was possible, WSELs for the detailed study streams were compared to the Effective WSELs (see Table 7). The differences between the restudy and Effective WSELs existed primarily due to differences in the flow values, topographic data, or land use changes between the restudy and Effective study. For cross sections near the downstream limit of study, the Effective WSELs may also be affected by backwater from the main stem.

Locations with a difference in WSEL greater than 3 feet between the new and Effective studies were reviewed for modeling issues and anomalies. For each difference greater than 3 feet, a possible explanation for the change is provided in Table 7.

Comparison of the floodplain extents between restudy and Effective suggested very similar 1- percent-annual-chance event floodplain boundaries. Even though there were differences between WSELs, the overbank cross-section geometry typically had a steep slope near the end of floodplain limits, which produced similar floodplain boundaries in spite of WSEL differences.

Table 7: Comparison of Effective and New WSELs (NAVD88)

| Stream | Effective Cross Section Location | Effective WSEL (Feet) | Closest Model Cross Section | New WSEL (Feet) | Delta (Feet) | Explanation (if greater than 3 feet difference) |
| --- | --- | --- | --- | --- | --- | --- |
| Deckers Creek | A | 813.8 | 485.02 | 814.1* | 0.2 | Backwater |
| Deckers Creek | B | 814.4 | 2203.6 | 814.1* | -0.3 | Effective elevation with backwater and co-incident peak |
| Deckers Creek | C | 817.6 | 2915.59 | 814.9 | -2.7 |  |
| Deckers Creek | D | 820.1 | 3838.58 | 817.0 | -3.1 | Model calibrated to gage rating curve |
| Deckers Creek | E | 825.6 | 5189.62 | 821.9 | -3.7 | Model calibrated to gage rating curve |
| Deckers Creek | F | 830.6 | 6469.79 | 830.4 | -0.3 |  |
| Deckers Creek | G | 837.4 | 9078.49 | 838.1 | 0.7 |  |
| Deckers Creek | H | 845.6 | 11587.01 | 844.1 | -1.5 |  |
| Deckers Creek | I | 851.2 | 13149.25 | 850.3 | -0.9 |  |
| Deckers Creek | J | 861.5 | 15755.6 | 862.8 | 1.3 |  |
| Deckers Creek | K | 871.0 | 16968.09 | 867.8 | -3.2 | Differences in topographic data, updated survey, and flow values |
| Deckers Creek | L | 878.6 | 18557.19 | 877.7 | -0.9 |  |
| Deckers Creek | M | 892.2 | 20450.61 | 893.3 | 1.1 |  |
| Deckers Creek | N | 914.5 | 24391.41 | 916.2 | 1.7 |  |
| Deckers Creek | O | 940.4 | 27768.36 | 939.9 | -0.5 |  |
| Deckers Creek | P | 957.4 | 29587.63 | 957.6 | 0.2 |  |
| Dents Run | A | 812.8 | 488 | 812.8* | 0.0 |  |
| Dents Run | B | 812.8 | 2283 | 812.8* | 0.0 |  |
| Dents Run | C | 812.8 | 3049 | 812.8* | 0.0 |  |
| Dents Run | D | 815.1 | 4296 | 814.5 | -0.6 |  |
| Dents Run | E | 817.6 | 5813 | 820.5 | 2.9 |  |
| Dents Run | F | 820.8 | 6040 | 832.3 | 11.5 | Steep slope of WSEL profile near structure, higher restudy flows, updated topography |
| Dents Run | G | 825.4 | 7245 | 832.9 | 7.5 | Higher restudy flows, updated topography |
| Dents Run | H | 830.6 | 7827 | 834.1 | 3.5 | Higher restudy flows, updated topography |
| Hickman Run | A | 871.2 | 166 | 871.2* | 0.0 |  |
| Hickman Run | B | 890.0 | 1667 | 891.2 | 1.2 |  |
| Hickman Run | C | 911.1 | 2537 | 913.6 | 2.5 |  |
| Hickman Run | D | 915.3 | 3038 | 915.2 | -0.1 |  |
| Hickman Run | E | 921.8 | 4073 | 923.7 | 1.9 |  |
| Hickman Run | F | 932.5 | 4534 | 938.0 | 5.5 | Culvert and overland modeling are different, higher restudy flows, updated topography |
| Hickman Run | G | 940.3 | 5353 | 947.9 | 7.6 | Culvert and overland modeling are different, higher restudy flows, updated topography |
| Hickman Run | H | 961.4 | 5963 | 955.4 | -6.0 | Culvert and overland modeling are different, higher restudy flows, updated topography |
| Hickman Run | I | 965.9 | 6638 | 957.4 | -8.5 | Culvert and overland modeling are different, higher restudy flows, updated topography |
| Hickman Run | J | 966.0 | 7675 | 958.2 | -7.8 | Culvert and overland modeling are different, higher restudy flows, updated topography |
| Hickman Run | K | 966.1 | 7918 | 958.6 | -7.5 | Culvert and overland modeling are different, higher restudy flows, updated topography |
| Hickman Run | L | 966.2 | 8531 | 963.6 | -2.6 |  |
| Hickman Run | M | 966.3 | 8986 | 964.1 | -2.2 |  |
| Hickman Run | N | 966.3 | 9190 | 965.8 | -0.5 |  |
| Hickman Run | O | 969.6 | 9995 | 970.7 | 1.1 |  |
| Pompano Run | A | 843.7 | n/a | n/a | n/a | The flow is completely contained in the underground pipe. |
| Pompano Run | B | 862.3 | 1580 | 874.2 | 11.9 | Differences in topographic data, updated survey, and flow values. Elevations upstream of the road based on HMS, which simulated storage routing. |

* Elevation with consideration of backwater effects

Comparison of the restudy and Effective floodway showed a narrower restudy floodway for Deckers Creek, and several of buildings were removed from the Effective floodway. For Hickman Run, portions of the restudy floodway were wider than the Effective floodway; however, the restudy included fewer building in the floodway compared to the Effective. The Effective floodway for Hickman Run appears to be shifted compared to current topography. For Dents Run the Effective and restudy floodways have similar widths, and the restudy did not add any additional buildings to the floodway compared to the Effective floodway. For Pompano Run, the stream is now piped underground downstream of Monongahela Boulevard, so the comparison of the floodways was not performed. Burroughs Run, Reuben Run, Pompano Run (upstream portion), and Scratchers Run had no Effective floodways. The comparison of restudy and Effective floodway widths is presented in Table 8.

Table 8: Comparison of Floodway Widths

| Stream | Effective Cross Section Location | Effective Floodway Width (Feet) | Closest Model Cross Section | New Floodway Width (Feet) | Delta (Feet) |  |
| --- | --- | --- | --- | --- | --- | --- |
| Deckers Creek | A | 70 | 485.02 | 77 |  | 7 |
| Deckers Creek | B | 93 | 2203.6 | 60 |  | -33 |
| Deckers Creek | C | 74 | 2915.59 | 82 |  | 8 |
| Deckers Creek | D | 83 | 3838.58 | 58 |  | -25 |
| Deckers Creek | E | 100 | 5189.62 | 83 |  | -17 |
| Deckers Creek | F | 100 | 6469.79 | 105 |  | 5 |
| Deckers Creek | G | 130 | 9078.49 | 149 |  | 19 |
| Deckers Creek | H | 150 | 11587.01 | 217 |  | 67 |
| Deckers Creek | I | 205 | 13149.25 | 169 |  | -36 |
| Deckers Creek | J | 286 | 15755.6 | 85 |  | -201 |
| Deckers Creek | K | 100 | 16968.09 | 54 |  | -46 |
| Deckers Creek | L | 90 | 18557.19 | 92 |  | 2 |
| Deckers Creek | M | 251 | 20450.61 | 110 |  | -141 |
| Deckers Creek | N | 179 | 24391.41 | 218 |  | 39 |
| Deckers Creek | O | 345 | 27768.36 | 180 |  | -165 |
| Deckers Creek | P | 65 | 29587.63 | 51 |  | -14 |
| Dents Run | A | 120 | 488 | 130 |  | 10 |
| Dents Run | B | 120 | 2283 | 122 |  | 2 |
| Dents Run | C | 70 | 3049 | 160 |  | 90 |
| Dents Run | D | 90 | 4296 | 89 |  | -1 |
| Dents Run | E | 92 | 5813 | 47 |  | -45 |
| Dents Run | F | 62 | 6040 | 56 |  | -6 |
| Dents Run | G | 43 | 7245 | 45 |  | 2 |
| Dents Run | H | 41 | 7827 | 39 |  | -2 |
| Hickman Run | A | 39 | 166 | 71 |  | 32 |
| Hickman Run | B | 35 | 1667 | 27 |  | -8 |
| Hickman Run | C | 39 | 2537 | 91 |  | 52 |
| Hickman Run | D | 65 | 3038 | 70 |  | 5 |
| Hickman Run | E | 37 | 4073 | 31 |  | -6 |
| Hickman Run | F | 66 | 4534 | 71 |  | 5 |
| Hickman Run | G | 38 | 5353 | 73 |  | 35 |
| Hickman Run | H | 20 | 5963 | 21 |  | 1 |
| Hickman Run | I | 40 | 6638 | 214 |  | 174 |
| Hickman Run | J | 59 | 7675 | 231 |  | 172 |
| Hickman Run | K | 40 | 7918 | 163 |  | 123 |
| Hickman Run | L | 130 | 8531 | 91 |  | -39 |
| Hickman Run | M | 90 | 8986 | 39 |  | -51 |
| Hickman Run | N | 120 | 9190 | 26 |  | -94 |
| Hickman Run | O | 100 | 9995 | 55 |  | -45 |
| Pompano Run | A | 52 | n/a | n/a |  | n/a |
| Pompano Run | B | 72 | 1580 | 110 |  | 38 |

## Discussion of Significant Differences in WSELs (>5 feet)

Significant differences in WSELs were observed along portions Dents Run, Hickman Run, and Pompano Run. The differences in WSELs were attributed to differences in peak discharges, topographic data, development in the study area, and advancement in modeling techniques after the Effective study.

Peak Discharges

The peak discharges for Dents Run, Hickman Run, and Pompano Run were computed using regression equations for West Virginia, National Urban Equations, and the HEC-HMS model, respectively. The updated hydrologic analysis showed a significant increase in the peak discharges compared to the Effective study, which was attributed to the WSEL increases. The Effective and restudy peak discharge comparison is shown in Table 9.

Table 9: Effective vs. Restudy Peak Discharge Comparison

| Flooding Source | Location | Effective Discharges |  |  |  |  | Restudied Discharges |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Drainage Area (sq. mi.) | 10% | 2% | 1% | 0.2% | Drainage Area (sq. mi.) | 10% | 2% | 1% | 0.2% |
| Dents Run | Confluence with Monongahela River | 14.3 | 1,758 | 2,526 | 2,917 | 3,785 | 14.2 | 2,109 | 3,304 | 3,864 | 4,916 |
|  | Town of Granville corporate limits | 11.5 | 1,562 | 2,260 | 2,613 | 3,408 | 12.05 | 1,808 | 2,863 | 3,348 | 4,274 |
| Hickman Run | Confluence with Monongahela River | 2.6 | 544 | 929 | 1,120 | 1,790 | 2.67 | 888 | 1,352 | 1,575 | 2,005 |
| Pompano Run | Limit of detailed study | 3.16 | 604 | 921 | 1,079 | 1,464 | 1.69 | 1,093 | 1,709 | 1,967 | 2,605 |

Topographic Data

The restudy utilized the latest LiDAR data from WVDEP to create HEC-RAS model geometries. Additionally, as part of this restudy, structure and natural cross-section survey data for Dents Run, Hickman Run, and Pompano Run was collected in April 2015. The restudy topographic data is more accurate than the Effective topographic data and represent the land use changes that occurred after Effective study. For Dents Run and Pompano Run, the 5-foot contour data developed in the 1960s was used for the Effective study. For Hickman Run, the 4-foot contour data developed in the 1980s was used for the Effective study.

The large differences in the WSELs along Dents Run, Hickman Run, and Pompano Run can be justified, as the restudy used more accurate and the latest hydrologic and topographic information, and the information was analyzed using advanced hydraulic modeling techniques compared to the Effective study.

Dents Run

At the lettered cross sections F and G on Dents Run for the Effective study, differences between the Effective and new studies range from 3.5 to 11.5 feet. After analysis of these large WSEL differences, it was concluded that the topography used in the new study is more accurate than the topography used in the Effective study. Recent survey data collected along Dents Run compares well with the new topography and did not match the ground inverts on the Effective FIS profile. Additionally, the new study for Dents Run has higher peak discharges than the Effective study. Since updated and more accurate information was used in the restudy, the differences in WSELs can be justified.

Hickman Run

At the lettered cross sections F through K on Hickman Run for the Effective study, differences between the Effective and new studies range from -8.5 to 7.6 feet. The large differences in WSELs can primarily be attributed to different culvert and overland modeling methods, as well as higher peak discharges. This restudy also uses updated LiDAR data and surveyed data collected as part of this restudy for creating the model geometry. The difference in the restudy and Effective study can be justified, as the restudy used more accurate and more recent information and was analyzed using advanced modeling techniques compared to the Effective study.

Pompano Run

At the lettered cross section B for the Effective study, a difference of 11.9 feet was observed on Pompano Creek. Further investigation of the area showed significant development in the area (based on Google Earth historical aerials) after the Effective study, and as a result, topography has completely changed. Additionally, the stream is now piped underground downstream of Monongahela Boulevard and outlets at the confluence with Monongahela River, which was not the case at the time of the Effective study. Comparison of Monongahela Boulevard top of road elevation and upstream culvert invert showed a difference of 8 to 10 feet. The restudy used updated LiDAR data along with surveyed elevations to create the model geometry. Additionally, the WSELs upstream of the road were determined from HEC-HMS storage routing. The difference in the restudy and Effective study can be justified, as the restudy used more accurate and more recent hydrologic and topographic information and was analyzed using advanced modeling techniques compared to the Effective study.

## Special Considerations

Long Culverts

Many of the study streams are located in heavily urbanized areas, and there are instances where long culvert systems exist with lengths greater than 500 feet. Generally the inlet and outlet inverts and sizes were available from the survey data collected for the study. However, for the underground portion, no construction plans showing culvert alignments, size changes, and slope changes were available. Therefore, a constant slope with inlet and outlet dimensions was used for the culvert capacity calculations. The culvert capacity calculations were performed using Bentley CulvertMaster Version 3.3 or the Federal Highway Administration HY-8 Culvert Hydraulic Analysis Program, Version 7.2. The lowest top of road elevation was entered in the program as the headwater elevation to obtain the maximum flow carrying capacity of the culvert before road overtopping would occur. For culverts that were large enough to contain all flow frequencies, the floodplain and floodway were truncated over the length of the culvert to show flow containment within the culvert. For culverts that contained a portion of the flow, the culvert capacity was deducted from the total flow, and the overland portion was modeled. For the overland model, the culvert was removed from the HEC-RAS models to allow overland cross sections to be placed. The cross sections were added over the length of the culvert to capture WSELs and the floodplain from flows overtopping the culvert. The floodplain and floodway resulting from the overland model were mapped over the culvert to reflect the final flood hazard area.

Hickman Run

For Hickman Run, two long underground culverts were modeled through culvert capacity analysis and overland modeling analysis. The culvert capacity was determined using HY-8, Version 7.2. The calculated overtopping discharges were modeled as overland flow in HEC-RAS to obtain the overland floodplain mapping. The overland cross sections for the culverts modeled in this manner are cross sections 5353 through 8116 (Figure 3) and cross sections 9533 through 9966 (Figure 4).

Figure 3: Hickman Run Culvert from Cross Sections 5353 to 8116

Figure 4: Hickman Run Culvert under State Route 310

Pompano Run

There are three long culvert systems within the study reach of Pompano Run. Two culvert systems underneath the West Virginia University football field (Figure 5) and one underneath Monongahela Boulevard (Figure 6) all contain the 100-year flow. Accordingly, the floodplain-floodway boundaries over the culverts were truncated to show flow containment within the culverts.

For approximately 1,200 feet along the culvert system starting at University Boulevard and ending near Takoma Street (Figure 7), an overland flow was modeled. The culvert analysis performed using CulvertMaster showed a capacity of 859 cubic feet per second (cfs), which was deducted from total 1-percent-annual-chance peak flow of 1,139 cfs to obtain the flow of 280 cfs for the overland portion. The overland portion of the model did not include the culvert, and 280 cfs was applied in the HEC-RAS model. Just downstream of the culvert outfall, the total flow of 1,300 cfs (slightly higher than the upstream end) was applied in the model.

Additionally, the WSELs were set upstream of Monongahela Boulevard and University Boulevard based the HEC-HMS model, where storage routings were modeled.

Figures 5, 6, and 7 show the floodway and 1-percent-annual-chance floodplain for each area, as well as streamlines, cross sections, and topographic contours.

Figure 5: Floodplains in the Vicinity of Mountaineer Field

Figure 6: Floodplains in the Vicinity of Monongahela Boulevard

Figure 7: Floodplains in the Vicinity of University Boulevard and Takoma Street

# Special Problems

See Section 3.5 Special Considerations, for stream-specific issues and resolutions. No other special problems were encountered during the hydraulic analyses.

# Exceptions

The spatial files “S_Levee” and “S_RiverMark” are not included because they were not applicable for this study.

# Conclusions

This TSDN documents the WSEL values to be applied to the floodplain mapping in the Upper Monongahela Watershed study area in Marion and Monongalia Counties. The updated floodplain mapping will include the 1- and 0.2-percent-annual-chance events, and floodways for the updated detailed streams based on the WSELs determined from the hydraulic computations and analyses presented herein. Profile elevations will include the modeled frequencies for the 10-, 4-, 2-, 1-, 1+-, and 0.2-percent-annual-chance events. All hydraulic calculations and modeling meet FEMA specifications to be used for FIRM production.

A complete set of spatial files, model input, and output files, where applicable, were developed in accordance with FEMA’s FIRM Database Technical Reference and Data Capture Standards Technical Reference (FEMA, 2014A and 2014B). This set of data was loaded to the MIP with a digital copy of this report.

# References

Chow, V.T., 1959. Open-channel hydraulics. McGraw-Hill, New York. Page 113, Table 5-6, Section D.

FEMA, 2015A. Field Survey Technical Support Data Notebook, Task Order HSFE03-14-J-0016 for Upper Monongahela Watershed, WV, prepared by RAMPP. Washington D.C., May 31, 2015.

FEMA, 2015B. Hydrologic Analysis Technical Support Data Notebook, Task Order HSFE03-14-J-0016 for Upper Monongahela Watershed HUC 8 05020003: Marion and Monongalia Counties, WV, prepared by RAMPP. Washington, D.C., August 21, 2015.

FEMA, 2015C. Topographic Data Development Technical Support Data Notebook, Task Order HSFE03-14-J-0016 for Upper Monongahela Watershed, prepared by RAMPP. Washington, D.C., April 30, 2015.

FEMA, 2014A. FIRM Database Technical Reference, Preparing Flood Insurance Rate Map Databases. Washington, D.C., November 2014.

FEMA, 2014B. Data Capture Technical Reference. Washington, D.C., November 2014.

FEMA, 2012. Flood Insurance Study, Marion County, West Virginia and Incorporated Areas. Washington, D.C., June 19, 2012.

FEMA, 2010. Flood Insurance Study, Monongalia County, West Virginia and Incorporated Areas. Washington, D.C., January 20, 2010.

FEMA, 2003. Guidelines and Specifications for Flood Hazard Mapping Partners, as amended. Washington, DC.

Morgantown Utility Board, 2005. Drainage Study – Burroughs and Pompano Runs, March 2005.

USACE, 2010. HEC-RAS Version 4.1 Hydraulic Reference Manual. Hydrologic Engineering Center, Davis, California, January 2010.

APPENDIX A

TSDN Documents

| M-1 TSDN INVENTORY FORM |  |  |
| --- | --- | --- |
| TSDN CATEGORY | DATA TYPE | DATA SUBMITTED |
| General Documentation | Special Problem Reports Index | N/A |
|  | Special Problem Reports | N/A |
|  | Contact Reports Index | N/A |
|  | Contact Reports | N/A |
|  | Meeting Minutes/Reports Index | N/A |
|  | Meeting Minutes/Reports | N/A |
|  | Correspondence with/from FEMA | Yes |
|  | Correspondence with/from Contractor | N/A |
|  | Other General Correspondence | N/A |
| Engineering Analyses | Hydrologic Analyses Index | N/A |
|  | Summary Report of Hydrologic Analyses | N/A |
|  | Computer Models, Calculations, and Execution Runs | N/A |
|  | Summary Report for Independent QA/QC Review of Hydrologic Analyses | N/A |
|  | Hydraulic Analyses Index | Yes |
|  | Cross Section Information | Yes |
|  | Floodway Analyses | Yes |
|  | Key To Cross Section Labeling | Yes |
|  | Computer Models, Calculations, and Execution Runs | Yes |
|  | Cross-Section Plots | Yes |
|  | Computer Models, Calculations, and Execution Runs | Yes |
|  | Summary Report for Independent QA/QC Review of Riverine Hydraulic Analyses and Floodplain Mapping | Yes |
|  | Key To Transect Labeling | N/A |
|  | Transect and Surge Data | N/A |
|  | Wave Height Information | N/A |
|  | Computer Models, Calculations, and Execution Runs | N/A |
|  | Summary Report for Independent QA/QC Review of Coastal Flooding Analyses and Floodplain Mapping | N/A |
|  | Shallow Flooding Models, Calculations, and Execution Runs | N/A |
|  | Summary Report for Independent QA/QC Review of Shallow Flooding Analyses and Floodplain Mapping | N/A |
|  | Ice-Jam Flooding Models, Calculations, and Execution Runs | N/A |
|  | Summary Report for Independent QA/QC Review of Ice-Jam Flooding Analyses and Floodplain Mapping | N/A |
|  | Shallow Flooding Models, Calculations, and Execution Runs | N/A |
|  | Summary Report for Independent QA/QC Review of Shallow Flooding Analyses and Floodplain Mapping | N/A |
|  | Ice-Jam Flooding Models, Calculations, and Execution Runs | N/A |
|  | Summary Report for Independent QA/QC Review of Ice-Jam Flooding Analyses and Floodplain Mapping | N/A |
|  | Alluvial Fan Flooding Models, Calculations, and Execution Runs | N/A |
|  | Summary Report for Independent QA/QC Review of Alluvial Fan Flooding Analyses and Floodplain Mapping | N/A |

| M-7. MAPPING INFORMATION INDEX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Community Name: |  | Marion County, Monongalia County |  |  |  | State: West Virginia |  |  |  |
| Community ID No.: |  | 54061C, 54049C |  |  |  |  |  |  |  |
| Compiled By: |  | RAMPP |  |  |  |  |  |  |  |
| Date TSDN Submitted: |  | May 2016 |  |  |  |  |  |  |  |
| Type/Purpose of Map | Date | Paper Copy |  |  | Electronic Media |  |  |  |  |
|  |  | No. of Sheets | Exhibit No. |  | File Type |  | File Name | Projection | Exhibit No. |
| Base Flood Elevations | 1/29/2016 |  |  |  | Shape file (*.dbf, *.prj, *.shp, *.shx) |  | S_BFE | NAD 1983 UTM Zone 17N | MIP Submittal |
| Flood Hazard Areas | 1/29/2016 |  |  |  | Shape file (*.dbf, *.prj, *.shp, *.shx) |  | S_FLD_HAZ_AR | NAD 1983 UTM Zone 17N | MIP Submittal |
| General Structures (bridges, culverts, dams) | 1/29/2016 |  |  |  | Shape file (*.dbf, *.prj, *.shp, *.shx) |  | S_GEN_STRUCT | NAD 1983 UTM Zone 17N | MIP Submittal |
| Profile Baseline | 1/29/2016 |  |  |  | Shape file (*.dbf, *.prj, *.shp, *.shx) |  | S_PROFILE_BASELINE | NAD 1983 UTM Zone 17N | MIP Submittal |
| Start station location for profiles | 1/29/2016 |  |  |  | Shape file (*.dbf, *.prj, *.shp, *.shx) |  | S_STN_START | NAD 1983 UTM Zone 17N | MIP Submittal |
| Project area boundary | 1/29/2016 |  |  |  | Shape file (*.dbf, *.prj, *.shp, *.shx) |  | S_SUBMITTAL_INFO | NAD 1983 UTM Zone 17N | MIP Submittal |
| HEC-RAS Cross Sections | 1/29/2016 |  |  |  | Shape file (*.dbf, *.prj, *.shp, *.shx) |  | S_XS | NAD 1983 UTM Zone 17N | MIP Submittal |
| Cross Sections Table | 1/29/2016 |  |  |  | *.dbf |  | L_XS_ELEV | N/A | MIP Submittal |
| Manning’s “n” Table | 1/29/2016 |  |  |  | *.dbf |  | L_ManningsN | N/A | MIP Submittal |
| Source Citation | 1/29/2016 |  |  |  | *.dbf |  | L_Source_CIT | N/A | MIP Submittal |

NAD 1983 = North American Datum of 1983

APPENDIX B

Hydraulic Analysis Supporting Data

All hydraulic analysis data are included on MIP submittal folder

Table B.1: Structures with Field Survey Data

| Stream | Structure | Field Survey ID |
| --- | --- | --- |
| Deckers Creek | University Avenue | DEC_01X |
| Deckers Creek | Rogers Street | DEC_02 |
| Deckers Creek | Pedestrian Bridge | DEC_03 |
| Deckers Creek | U.S. Route 68 | DEC_06X |
| Dents Run | Driveway | DEN_02 |
| Dents Run | Driveway | DEN_03 |
| Hickman Run | Access Road for Fairmont Sewer | HIC_01 |
| Hickman Run | Access Road for Fairmont Sewer | HIC_01A |
| Hickman Run | Morgantown Avenue | HIC_02 |
| Pompano Run | Monongahela Boulevard | POP_02 |
| Pompano Run | Chipps Hollow Road | POP_03 |
| Pompano Run | Footbridge at Krepps Park | POP_05 |
| Pompano Run | Laurel Street | POP_06 |
| Pompano Run | Patteson Drive | POP_07 |
| Pompano Run | Chik-Fil-A Driveway | POP_08 |
| Pompano Run | Raymond Park Footbridge | POP_10 |
| Pompano Run | WVU Alumni Building Crossing | POP_11 |
| Pompano Run | WVU Footbridge | POP_12 |
| Pompano Run | Football Stadium | POP_13 |
| Pompano Run | Willowdale Road | POP_14 |
| Pompano Run | Hoffman Avenue | POP_15 |
| Pompano Run | Bradley Street | POP_16 |
| Reuben Run | Driveway | REU_01 |
| Reuben Run | Bunner Ridge Road | REU_02 |
| Reuben Run | Pedestrian Bridge | REU_03 |
| Reuben Run | Driveway | REU_04 |
| Reuben Run | Private Road | REU_05 |
| Reuben Run | Private Bridge | REU_06 |
| Scratchers Run | Driveway | SCR_01 |
| Scratchers Run | Firehouse Road | SCR_02 |
| Scratchers Run | Driveway | SCR_03 |

Table B.2: Structures Visited during Field Reconnaissance

| Report ID | Field ID | Approximate Coordinates | Stream Name | Road Name | Type |
| --- | --- | --- | --- | --- | --- |
| 428195_Field_Recon - Woodland Drive | Burr_01 | 39.666033, -79.976413 | Burroughs Run | Woodland Drive | Arch Bridge |
| 428202_Field_Recon - Valley Road | Burr_02 | 39.663734, -79.974868 | Burroughs Run | Valley Road | Arch Bridge |
| 428206_Field_Recon - Martin Street | Burr_03 | 39.662971, -79.974870 | Burroughs Run | Martin Street | Arch Bridge |
| 428210_Field_Recon - Meridan Street | Burr_04 | 39.662216, -79.974736 | Burroughs Run | Meridan Street | Arch Bridge |
| 428219_Field_Recon - Aspen Street | Burr_05 | 39.661538, -79.974290 | Burroughs Run | Aspen Street | Arch Bridge |
| 428229_Field_Recon - Eastern Avenue | Burr_06 | 39.661676, -79.971910 | Burroughs Run | Eastern Avenue | Arch Bridge |
| 428233_Field_Recon - Killarney Drive | Burr_07 | 39.659682, -79.968806 | Burroughs Run | Killarney Drive | Bridge and Circular culvert |
| 428246_Field_Recon - Dogwood Avenue | Burr_08 | 39.659729, -79.968965 | Burroughs Run | Dogwood Avenue | Circular |
| 428253_Field_Recon - Corvet Avenue | Burr_09 | 39.659093, -79.968025 | Burroughs Run | Corvet Avenue | Bridge |
| 428263_Field_Recon - Pedestrian Bridge | Burr_10 | 39.659149, -79.966550 | Burroughs Run | Pedestrian Bridge | Arch Bridge |
| 428273_Field_Recon - removed berm | Burr_11 | 39.657920, -79.964270 | Burroughs Run | Berm (removed) | Berm (removed) |
| 427684_Field_Recon - Footbridge | Deck_01 | 39.628228, -79.960838 | Deckers Creek | Footbridge | Bridge |
| 427953_Field_Recon - University Avenue | Deck_02 | 39.627513, -79.960155 | Deckers Creek | University Avenue | Bridge |
| 427965_Field_Recon - South High Street | Deck_02B | 39.626287, -79.958971 | Deckers Creek | South High Street | Bridge |
| 427975_Field_Recon - Pleasant Street | Deck_03 | 39.627351, -79.955426 | Deckers Creek | Pleasant Street | Bridge |
| 427979_Field_Recon - West Virginia 7 | Deck_04 | 39.628105, -79.953742 | Deckers Creek | West Virginia 7 | Bridge |
| 427985_Field_Recon - Dam | Deck_ADD_01 | 39.629308, -79.952922 | Deckers Creek | Dam | Dam/Weir |
| 427997_Field_Recon - Dam | Deck_ADD_02 | 39.625037, -79.945341 | Deckers Creek | Dam | Dam/Weir |
| 428004_Field_Recon - Rogers Ave | Deck_05 | 39.624872, -79.944843 | Deckers Creek | Rogers Avenue | Bridge |
| 428063_Field_Recon - Footbridge | Deck_ADD_03 | 39.628202, -79.939413 | Deckers Creek | Footbridge | Bridge |
| 428073_Field_Recon - White Avenue | Deck_05_Real | 39.628818, -79.934525 | Deckers Creek | White Avenue | Arch Bridge & Circular Culvert |
| 428083_Field_Recon - Greenbag Road | Deck_06 | 39.626206, -79.930441 | Deckers Creek | Greenbag Road | Bridge |
| No structure exists at this location | Deck_07 | Removed from Scope | Deckers Creek | N/A | N/A |
| 428102_Field_Recon - Footbridge | Deck_08 | 39.621734, -79.925617 | Deckers Creek | Footbridge | Bridge |
| 428143_Field_Recon - Carnegie Street | Deck_09 | 39.618223, -79.924962 | Deckers Creek | Carnegie Street | Bridge |
| 428153_Field_Recon - Earl L Core Road | Deck_10 | 39.608025, -79.919087 | Deckers Creek | Earl L. Core Road | Bridge |
| 428160_Field_Recon - Private Road | Deck_11 | 39.607978, -79.898192 | Deckers Creek | Private Road | Bridge |
| 428165_Field_Recon - Dam | Deck_ADD_04 | 39.607706, -79.897834 | Deckers Creek | Dam | Dam/Weir |
| 428170_Field_Recon - Earl L Core Road | Deck_12 | 39.608927, -79.889015 | Deckers Creek | Earl L. Core Road | Arch Bridge |
| 427665_Field_Recon - US Route 19 | Dent_01 | 39.642872, -79.983863 | Dents Run | U.S. Route 19 (Dunkard Avenue) | Bridge |
| 427660_Field_Recon - CSX Railroad | Dent_02 | 39.642279, -79.983872 | Dents Run | CXS Railroad | Bridge |
| Unable to visit this structure due to access issues | Dent_03 | 39.633923, -79.993271 | Dents Run | Country Road 49 (Dents Run Boulevard) | Unknown |
| 427423_Field_Recon - Railroad_removed from scope | Hick_01 | 39.485339, -80.119854 | Hickman Run | Railroad (removed) | Bridge (removed) |
| 427439_Field_Recon - Private Drive | Hick_02 | 39.485001, -80.119265 | Hickman Run | Private Drive | Circular Culvert |
| 427447_Field_Recon - Railroad_removed from scope | Hick_03 | 39.485441, -80.117682 | Hickman Run | Railroad (removed) | Bridge (removed) |
| 427476_Field_Recon - Access Road | Hick_04 | 39.486819, -80.110328 | Hickman Run | Access Road | Circular Culvert |
| Unable to visit this structure due to access issues | Hick_05 | 39.484565, -80.110529 | Hickman Run | Hess Court | Unknown |
| 427508_Field_Recon - King Street | Hick_06A | 39.479672, -80.116886 | Hickman Run | King Street | Square Culvert |
| 427514_Field_Recon - Culvert Under Building | Hick_06B | 39.479544, -80.117002 | Hickman Run | Culvert Under Building | Square Culvert |
| 427527_Field_Recon - Loading Docks | Hick_ADD_01 | 39.478612, -80.118259 | Hickman Run | Loading Docks | Bridge |
| 427532_Field_Recon - Private Road | Hick_07 | 39.478006, -80.119393 | Hickman Run | Private Road | Bridge |
| 427585_Field_Recon - Private Road_East Ratton Road | Hick_09 | 39.476678, -80.121908 | Hickman Run | Private Road East of Ratton Road | Circular Culvert/Square Culvert |
| 427595_Field_Recon - WV-310 | Hick_08 | 39.476889, -80.121425 | Hickman Run | West Virginia 310 | Square Culvert |

APPENDIX C

Quality Assurance / Quality Control

Quality Assurance and Quality Control were conducted following the RAMPP Quality Management Plan

APPENDIX D

Correspondence

APPENDIX E

CD/DVD with All Applicable Data

(All digital data loaded to the MIP)