5.2	Hydraulic Analysis

Hydraulic analyses were performed as a part of Upper Monongahela Watershed Study for approximately 15.2 miles of detailed level study. The streams studied by detailed methods included Burroughs Run, Deckers Creek, Dents Run, Hickman Run, Pompano Run, Reuben Run, and Scratchers Run. The analyses consisted of determining the water-surface elevations for the 10-, 2-, 1-, 0.2-percent-annual-chance and 1-percent-plus-annual-chance flood events and floodway analysis was also performed for streams studied by detailed methods.

The hydraulic models were developed using HEC-RAS Version 4.1 and the HEC-GeoRAS an ArcGIS extension (version 10.0). Cross sections were generated using HEC-GeoRAS and terrain data processed by RAMPP in 2015. The streamlines were generated using 2-foot contour data and were corrected using aerial photography. Manning’s n-value polygons for the studied streams were created using the aerial imagery and different land use values were determined using Manning’s n-values published by Chow V.T. in 1959 and field survey.

The downstream boundary conditions were set using the normal depth method unless the stream tied directly into an effective or restudied detailed study of the same stream, in which case the Known WSEL downstream boundary condition was used.

The following tabulation in Table 6, "Manning’s n-values”, shows the channel and overbank n-values for the streams studied by detailed methods in this countywide FIS.

For this FIS, locations of selected cross sections used in the hydraulic analyses are shown on the Flood Profiles (Exhibit 1).  For stream segments for which a floodway was computed (Section 4.2), selected cross section locations are also shown on the FIRM (Exhibit 2).  Flood profiles were drawn showing computed water-surface elevations for floods of the selected recurrence intervals.

### TABLE 6 – MANNING'S n-VALUES

| Stream | Channel “n” | Overbank “n” |
| --- | --- | --- |
| Burroughs Run | 0.03 – 0.045 | 0.03 – 0.12 |
| Deckers Creek | 0.038- 0.049 | 0.05-0.13 |
| Dents Run | 0.035 – 0.050 | 0.03 – 0.12 |
| Hickman Run | 0.040 – 0.050 | 0.03 – 0.13 |
| Pompano Run | 0.028- 0.055 | 0.06-0.13 |
| Reuben Run | 0.050 – 0.055 | 0.03 – 0.12 |
| Scratchers Run | 0.045 – 0.050 | 0.05 – 0.12 |