PROJECT NARRATIVE - HYDRAULICS

# Project Overview

This hydraulic study was conducted for the Department of Homeland Security’s Federal Emergency Management Agency (FEMA) as part of updating the Flood Insurance Studies (FIS) and Flood Insurance Rate Maps (FIRM) for Marion and Monongalia Counties in the Upper Monongahela Watershed. The study was performed by Risk Analysis, Mapping, and Planning Partners (RAMPP), under the Production and Technical Services contract HSFE03-14-J-0016. This analysis was performed in accordance with FEMA’s Guidelines and Specifications for Flood Hazard Mapping Partners, as amended (G&S).

The Upper Monongahela Watershed (Hydrologic Unit Code (HUC)-8 05020003) covers 7,340 square miles in southwest Pennsylvania and northern West Virginia. The majority of the watershed is located in West Virginia in Marion, Monongalia, Preston, Taylor, and Wetzel Counties. The remaining portion of the watershed is located in Fayette and Green Counties in western Pennsylvania. The horizontal projection used for this study was Universal Transverse Mercator (UTM) Zone 17N in meters. The vertical datum used for this study was North American Vertical Datum of 1988 (NAVD88).

Hydraulic computations and analyses for Upper Monongahela Watershed consisted of determining water surface elevations (WSELs) for the 10%, 4%, 2%, 1%, 1%, and 0.2% annual-chance, as well as a floodway analysis, for a total of 23.2 miles of detailed study reaches.

# 2.0	Project Work Scope

The following is the RAMPP's Scope of Work for Task Order HSFE03-14-J-0016 for Upper Monongahela Watershed, WV:

Scope: RAMPP performed hydraulic analyses for approximately 15.2 miles of Effective Zone AE. The modeling included the 10%, 4%, 2%, 1%, 1%+, and 2% annual-chance events based on peak discharges computed under Hydrologic Analyses. The hydraulic methods used for this analysis included enhanced level hydraulic modeling. The enhanced level included field surveys, floodways, and the 10%, 4%, 2%, 1%, 1%+, and 2% annual-chance events.

RAMPP used the cross-section and field data collected during Field Survey and the topographic data collected during the Topographic Data Collection, when appropriate, to perform the hydraulic analyses. The hydraulic analyses will be used to establish flood elevations and regulatory floodways for the subject flooding sources.

Standards: All Hydraulic Data work was be performed in accordance with the FEMA standards. RAMPP performed internal QA/QC reviews for all hydraulics, as well as perform external QA/QC by a separate RAMPP entity.

Deliverables: In accordance with FEMA’s G&S, RAMPP has made the following products available to FEMA by uploading the digital data to the MIP. A metadata file that complies with the National Flood Insurance Program (NFIP) Metadata Profiles Specifications will accompany the uploaded compliant digital data. Additionally, the TSDN format described in FEMA’s G&S has been delivered.

- Digital profiles of the 10%, 4% (except in PA), 2%, 1%, 1%+, and 0.2% annual chance events, representing existing conditions using the FEMA RASPLOT program or similar software

- Metadata file in the 2013 metadata specification

- Digital Floodway Data Tables for each flooding source, exportable via RASPLOT 3.0

- Digital hydraulic modeling (input and output) files

- Digital tables with range of Manning’s “n” values

- Explanations for unresolved messages from the CHECK-2 or CHECK-RAS program, as appropriate

- Digital versions of all backup data used in the analyses

- Digital versions of draft text for inclusion in the FIS report, in editable format (.doc or .docx)

- Deliverables include all input and output data, GIS data layers, and final

- TSDN Report uploaded to the MIP

  - A summary that describes the methodologies used, input and outputs, and provides the results of all automated or manual QA/QC review steps taken during the preparation of the FIRM as outlined in the approved QA/QC Plan

  - Where paper documentation is required by State Law for Professional certifications, you will submit the paper in addition to a scanned version of the paper for the digital record.

  - Appropriate leverage information, including who paid for the data and the amount of data used by the Flood Risk Project.

## 3.0	ISSUES

No issues or special problems were encountered during the hydraulics analysis.

# 4.0	Information for the Next Mapping partner

There is no specific information that would benefit the next mapping partner.