5.0 ENGINEERING METHODS

5.2 Hydraulic Analyses

Analyses of the hydraulic characteristics of flooding from the sources studied were carried out to provide estimates of the elevations of floods of the selected recurrence intervals. Base flood elevations on the FIRM represent the elevations shown on the Flood Profiles and in the Floodway Data tables in the FIS Report. Rounded whole-foot elevations may be shown on the FIRM in coastal areas, areas of ponding, and other areas with static base flood elevations. These whole-foot elevations may not exactly reflect the elevations derived from the hydraulic analyses. Flood elevations shown on the FIRM are primarily intended for flood insurance rating purposes. For construction and/or floodplain management purposes, users are cautioned to use the flood elevation data presented in this FIS Report in conjunction with the data shown on the FIRM. The hydraulic analyses for this FIS were based on unobstructed flow. The flood elevations shown on the profiles are thus considered valid only if hydraulic structures remain unobstructed, operate properly, and do not fail.

For this study, a one-dimensional steady state hydraulic models were developed using the USACE Hydraulic Engineering Center (HEC) River Analysis System (RAS) versions 5.0.7 (HEC, 2019). The cross-section geometry was extracted from Light Detection and Ranging (LiDAR) data collected by Quantum Spatial, Inc. in 2016. Survey data was collected for the detailed studies in the spring of 2018 and included elevation data from the channel, channel centerline, edge of water, top of bank, toe of bank, face of abutment, bridge corners, low chord, high points, centerline of the bridge, piers, elevation shots, end of bridge and the road profile. For the detailed studies, the channel bathymetry was modified by incorporating inverts from the effective study (FEMA, 2012) and collected survey data and the survey data was used to inform the modeled structures.

A summary of the methods used in hydraulic analyses performed for this project is provided in Table 12. Roughness coefficients are provided in Table 13. Roughness coefficients are values representing the frictional resistance water experiences when passing overland or through a channel. They are used in the calculations to determine water surface elevations. Greater detail (including assumptions, analysis, and results) is available in the archived project documentation.

Table 12. Summary of Hydrologic and Hydraulic Analysis

| Flooding Source | Study Limits Downstream Limit | Study Limits Upstream Limit | Hydrologic Model or Method Used | Hydraulic Model or Method Used | Date Analyses Completed | Flood Zone on FIRM | Special Considerations |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Dry Creek 1 | Confluence with Howard Creek | Approximately 100 feet downstream of Tuckahoe Road | PeakFQ 17C Gage Analysis and Gage-Weighted Equations | HEC-RAS 5.0.7 | March 2020 | AE |  |
| Greenbrier River 2 | Approximately 10,000 feet downstream of Seneca Trail S | Approximately 9,000 feet upstream of Seneca Trail S | Gage Analysis weighted with Regional Regression Equations | HEC-RAS 5.0.7 | March 2020 | AE | Hydraulic model was calibrated to high water marks collected for 2016 event |
| Greenbrier River 3 | Approximately 21,000 feet downstream of Fort Spring Pike | Approximately 10,000 feet downstream of Seneca Trail S | Gage Analysis weighted with Regional Regression Equations | HEC-RAS 5.0.7 | March 2020 | AE |  |
| Greenbrier River 4 | Approximately 9,000 ft upstream of Seneca Trail S | Approximately 1,500 ft upstream of Interstate 64 | Gage Analysis weighted with Regional Regression Equations | HEC-RAS 5.0.7 | March 2020 | AE |  |
| Meadow River 3 | Approximately 6,000 feet upstream of Central Ave | Approximately 8,000 feet downstream of Tommy Hall Road | PeakFQ 17C Gage Analysis and Gage-Weighted Equations | HEC-RAS 5.0.7 | March 2020 | AE |  |
| Wades Creek 1 | Confluence with Howard Creek | Approximately 50 feet upstream of Interstate 64 | Gage Analysis weighted with Regional Regression Equations | HEC-RAS 5.0.7 | March 2020 | AE |  |
| ALUMRN | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| ANGLINSCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| ANTHONYCK | Downstream extent | Upstream extent | Gage Weighting | HEC-RAS 5.0.7 | March 2020 | A |  |
| ANTNYCKT1 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| ANTNYCKT2 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| ANTNYCKT2_1 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| ANTNYCKT3 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| BEARRN | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| BEAVERCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| BECKYRN | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| BEECHRN1 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| BEVERLYFK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| BIGCLEARCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| BLDRRN | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| BOGGSCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| BOGGSRN | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| BROADRN | Downstream extent | Upstream extent | HMS | HEC-RAS 5.0.7 | March 2020 | A |  |
| BROWNCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| BUFFALOCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| BURDETTECK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| BURNSRN | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| BURNSRNT1 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| CALLAHNBR | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| CLBRSNCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| CLBRSNCKT1 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| CLBRSNCKT2 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| CLBRSNCKT3 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| CLBRSNCKT4 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| COATSRN | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| COLDKNOBFK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| COLDSPRBR | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| DAVYRN | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| DODSONBR | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| DRYCK2 | Downstream extent | Upstream extent | HMS | HEC-RAS 5.0.7 | March 2020 | A |  |
| DRYRN | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| EAGLEBR | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| FLMINGRN | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| FLMINGRNT1 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| FLYNNCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| GRBRRV5 | Downstream extent | Upstream extent | Gage Weighting | HEC-RAS 5.0.7 | March 2020 | A |  |
| GRBRRVTR1 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| GRBRRVTR2 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| GRBRRVTR4 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| GRBRRVTR5 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| HARTSRN | Downstream extent | Upstream extent | HMS | HEC-RAS 5.0.7 | March 2020 | A |  |
| HOGCMPRN | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| HOMINYCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| HOWARDCK | Downstream extent | Upstream extent | HMS | HEC-RAS 5.0.7 | March 2020 | A |  |
| HOWCKTR1 | Downstream extent | Upstream extent | HMS | HEC-RAS 5.0.7 | March 2020 | A |  |
| HUGHRTCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| HUGHRTCKT1 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| HUNTERSRN | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| IMPRVMTBR | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| INDIANCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| JERCHODFT | Downstream extent | Upstream extent | HMS | HEC-RAS 5.0.7 | March 2020 | A |  |
| JERCHODFTT1 | Downstream extent | Upstream extent | HMS | HEC-RAS 5.0.7 | March 2020 | A |  |
| JOBKNOBBR1 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| JOBKNOBBR2 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| KINCAIDRN | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| KTCHNCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| KTCHNCKT1 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| KUHNBR | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| LAURELBR | Downstream extent | Upstream extent | HMS | HEC-RAS 5.0.7 | March 2020 | A |  |
| LAURELCK1 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| LAURELCK2 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| LAURELCK3 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| LAURELCK4 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| LAURELRN1 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| LAURELRN2 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| LITTLECK1 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| LITTLECK2 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| LTCLEARCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| LTLAURELCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| LTROARCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| LTSEWELLCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| MCMILLCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| MEADOWCK1 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| MEADOWCK2 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| MEADOWRV1 | Downstream extent | Upstream extent | Gage Weighting | HEC-RAS 5.0.7 | March 2020 | A |  |
| MEADOWRV4 | Downstream extent | Upstream extent | Gage Weighting | HEC-RAS 5.0.7 | March 2020 | A |  |
| MFANTNYCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| MIDDLEBR | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| MILLCK1 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| MILLCK1T1 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| MILLCK2 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| MLGNCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| MLGNCKTR3 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| MLGNCKTR4 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| MLGNCKTR5 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| MORRISFK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| MTHODSTBR | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| MUDDYCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| NFANTNYCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| NFKCHERRYRV | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| OLFLDBR | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| OTTERCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| PATTSNCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| PEASERBR | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| PINEYCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| PNTHRCMPCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| RCKCMPRN | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| RENICKCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| ROARINGCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| ROBBINSRN | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| ROCKYRN | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| SAMCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| SECONDCK | Downstream extent | Upstream extent | Gage Weighting | HEC-RAS 5.0.7 | March 2020 | A |  |
| SFBIGCLRCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| SFKCHERRYRV | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| SIMMSRN | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| SLBCMPRN | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| SLPHRLCKRN | Downstream extent | Upstream extent | HMS | HEC-RAS 5.0.7 | March 2020 | A |  |
| SLSHLCKRN | Downstream extent | Upstream extent | HMS | HEC-RAS 5.0.7 | March 2020 | A |  |
| SMKHSEBR | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| SNAKERN | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| SNKNGCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| SNKNGCKT1 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| SNKNGCKT2 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| SNKNGCKT3 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| SNKNGCKT4 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| SNODGRSRN | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| SPICERN | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| SPRINGCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| SPRINGCKT3 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| STONYRN | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| TOMSCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| TUCKHORN | Downstream extent | Upstream extent | HMS | HEC-RAS 5.0.7 | March 2020 | A |  |
| TUCKHORNT2 | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| TWOMILERN | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| WADESCK2 | Downstream extent | Upstream extent | HMS | HEC-RAS 5.0.7 | March 2020 | A |  |
| WILEYRN | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| WOLFPENCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |
| YOUNGSCK | Downstream extent | Upstream extent | Regression | HEC-RAS 5.0.7 | March 2020 | A |  |

Table 13. Roughness Coefficients

| Flooding Source | Channel “n” | Overbank “n” |
| --- | --- | --- |
| Dry Creek 1 | .035-.045 | .016- .12 |
| Greenbrier River 2 | .045-.045 | .05-.1 |
| Greenbrier River 3 | .035-.04 | .016- .12 |
| Greenbrier River 4 | .03-.04 | .016- .12 |
| Meadow River 3 | .04-0.04 | .016-.12 |
| Wades Creek 1 | .04-.045 | .012-.12 |
| ALUMRN | .045-.045 | .05-.12 |
| ANGLINSCK | .045-.045 | .1-.1 |
| ANTHONYCK | .045-.045 | .025-.12 |
| ANTNYCKT1 | .045-.045 | .06-.12 |
| ANTNYCKT2 | .045-.045 | .035-.12 |
| ANTNYCKT2_1 | .045-.045 | .05-.12 |
| ANTNYCKT3 | .045-.055 | .035-.12 |
| BEARRN | .04-.04 | .12-.12 |
| BEAVERCK | .045-.045 | .035-0.1 |
| BECKYRN | .04-.045 | .12-.12 |
| BEECHRN1 | .045-.045 | .1-.1 |
| BEVERLYFK | .045-.045 | .06-.1 |
| BIGCLEARCK | .025-.045 | .035-.12 |
| BLDRRN | .045-.045 | .1-.1 |
| BOGGSCK | .045-.045 | .016-.12 |
| BOGGSRN | .045-.045 | .06-.1 |
| BROADRN | .045-.045 | .06-.12 |
| BROWNCK | .045-.055 | .025-.1 |
| BUFFALOCK | .045-.045 | .045-.12 |
| BURDETTECK | .04-.045 | .016-.12 |
| BURNSRN | .045-.045 | 0.05-0.1 |
| BURNSRNT1 | .045-.045 | .1-.1 |
| CALLAHNBR | .045-.045 | .045-.1 |
| CLBRSNCK | .045-.045 | .05-.1 |
| CLBRSNCKT1 | .045-.045 | .05-.1 |
| CLBRSNCKT2 | .045-.045 | .05-.1 |
| CLBRSNCKT3 | .045-.045 | .1-.1 |
| CLBRSNCKT4 | .045-.045 | .013-.1 |
| COATSRN | .04-.04 | .016-.12 |
| COLDKNOBFK | .04-.04 | .016-.12 |
| COLDSPRBR | .045-.045 | .1-.1 |
| DAVYRN | .045-.045 | .06-.1 |
| DODSONBR | .045-.045 | .06-.1 |
| DryCK2 | .04-.045 | .016-.12 |
| DRYRN | .045-.045 | .05-.1 |
| EAGLEBR | .045-.045 | 0.045-0.12 |
| FLMINGRN | .045-.045 | .05-.12 |
| FLMINGRNT1 | .045-.06 | .045-.12 |
| FLYNNCK | .045-.045 | .05-.1 |
| GRBRRV5 | .04-.04 | .016-.12 |
| GRBRRVTR1 | .045-.045 | .025-.1 |
| GRBRRVTR2 | .045-.045 | .05-.1 |
| GRBRRVTR4 | .045-.045 | .02-.1 |
| GRBRRVTR5 | .045-.045 | .045-.1 |
| HARTSRN | .04-.04 | .016-.12 |
| HOGCMPRN | .045-.045 | .1-.1 |
| HOMINYCK | .045-.045 | .045-.12 |
| HOWARDCK | .045-.05 | .02-.12 |
| HOWCKTR1 | .04-.04 | .016-.12 |
| HUGHRTCK | .045-.045 | .05-.1 |
| HUGHRTCKT1 | .045-.045 | .05-.1 |
| HUNTERSRN | .04-.04 | .12-.12 |
| IMPRVMTBR | .04-.04 | .016-.12 |
| INDIANCK | .045-.045 | .05-.1 |
| JERCHODFT | .045-.045 | .05-.12 |
| JERCHODFTT1 | .045-.045 | .05-.1 |
| JOBKNOBBR1 | .045-.045 | .055-.1 |
| JOBKNOBBR2 | .045-.045 | .055-.1 |
| KINCAIDRN | .045-.045 | .1-.1 |
| KTCHNCK | .045-.045 | .05-.1 |
| KTCHNCKT1 | .045-.045 | .05-.1 |
| KUHNBR | .045-.045 | .05-.1 |
| LAURELBR | .045-.045 | .05-.1 |
| LAURELCK1 | .045-.045 | .02- .1 |
| LAURELCK2 | .045-.045 | .06-.12 |
| LAURELCK4 | .045-.045 | .055-.1 |
| LAURELRN1 | .045-.045 | .06-.1 |
| LAURELRN2 | .045-.045 | .06-.1 |
| LITTLECK1 | .045-.045 | .05-.12 |
| LITTLECK2 | .045-.045 | .055-.1 |
| LTCLEARCK | .045-.045 | .055-.1 |
| LTLAURELCK | .04-.04 | .016-.12 |
| LTROARCK | .045-.045 | .06-.1 |
| LTSEWELLCK | .035-.04 | .016-.12 |
| MCMILLCK | .045-.045 | .055-.1 |
| MEADOWCK1 | .045-.045 | .02-.12 |
| MEADOWCK2 | .04-.04 | .016-.12 |
| MEADOWRV1 | .035-.045 | .016-.12 |
| MEADOWRV4 | .045-.045 | .035-.12 |
| MFANTNYCK | .045-.045 | .025-.12 |
| MIDDLEBR | .045-.045 | .055-.1 |
| MILLCK1 | .045-.045 | .05-.12 |
| MILLCK1T1 | .045-.045 | .05-.1 |
| MILLCK2 | .045-.045 | .035-.1 |
| MLGNCK | .045-.045 | .05-.12 |
| MLGNCKTR3 | .045-.045 | .05-.12 |
| MLGNCKTR4 | .045-.045 | .05-.12 |
| MLGNCKTR5 | .045-.045 | .05-.1 |
| MORRISFK | .045-.055 | .05-.1 |
| MTHODSTBR | .045-.045 | .035-.12 |
| MUDDYCK | .045-.045 | .035-.12 |
| NFANTNYCK | .045-.045 | .05-.12 |
| NFKCHERRYRV | .04-.04 | .016-.12 |
| OLFLDBR | .045-.045 | .05-.1 |
| OTTERCK | .045-.045 | .035-.1 |
| PATTSNCK | .045-.045 | .045-.1 |
| PEASERBR | .045-.045 | .07-0.12 |
| PINEYCK | .04-.04 | .1-.1 |
| PNTHRCMPCK | .045-.045 | .05-.1 |
| RCKCMPRN | .045-.045 | .055-.1 |
| RENICKCK | .045-.045 | .055-.1 |
| ROARINGCK | .045-.045 | .055-.1 |
| ROBBINSRN | .045-.045 | 0.05-0.12 |
| ROCKYRN | .04-.04 | .016-.12 |
| SAMCK | .045-.045 | .06-.1 |
| SECONDCK | .045-.045 | .035-.12 |
| SFBIGCLRCK | .045-.045 | .045-.1 |
| SFKCHERRYRV | .04-.04 | .06-.12 |
| SIMMSRN | .045-.045 | .05-.12 |
| SLBCMPRN | .045-.045 | .025-.12 |
| SLPHRLCKRN | .045-.045 | .05-.12 |
| SLSHLCKRN | .045-.045 | .05-.12 |
| SMKHSEBR | .045-.045 | .055-.1 |
| SNAKERN | .045-.045 | .035- .1 |
| SNKNGCK | .045-.045 | .02-.12 |
| SNKNGCKT1 | .045-.045 | .05-.1 |
| SNKNGCKT2 | .045-.045 | .05-.1 |
| SNKNGCKT3 | .045-.045 | .055-.1 |
| SNKNGCKT4 | .045-.045 | .05-.1 |
| SNODGRSRN | .045-.045 | .1-.1 |
| SPICERN | .045-.045 | .025- .12 |
| SPRINGCK | .045-.045 | .025- .12 |
| SPRINGCKT3 | .045-.045 | .035-.1 |
| STONYRN | .045-.045 | .05-.12 |
| TOMSCK | .045-.045 | .055-.12 |
| TUCKHORN | .04-.045 | .016-.12 |
| TUCKHORNT2 | .045-.045 | .05-.1 |
| TWOMILERN | .045-.045 | .06- .12 |
| WADESCK2 | .04-.04 | .016-.12 |
| WILEYRN | .045-.045 | .055-.12 |
| WOLFPENCK | .045-.045 | .02-.1 |
| YOUNGSCK | .045-.045 | .1-.12 |

References

Federal Emergency Management Agency, 2012. Flood Insurance Study, Greenbrier County, West Virginia and Incorporated Areas, Effective: October 16, 2012.

U.S. Army Corps of Engineers, Hydrologic Engineering Center (HEC), 2019. HEC-RAS River Analysis System, Version 5.0.7, U.S. Army Corps of Engineers (USACE).

West Virginia GIS Technical Center, 2011. Landuse/Landcover of West Virginia (2011), http://wvgis.wvu.edu/data/dataset.php?ID=451