Hydraulic Analysis Technical Support Data Notebook

Pendleton County, West Virginia

Prepared By:

Wood Environment & Infrastructure Solutions, INC.

4795 Meadow Wood Lane, Suite 310E

Chantilly, VA 20151

Federal Emergency Management Agency (FEMA) Region III

Cooperating Technical Partners Program 
Mapping Activity Statement No. WVDHSEM-2016-2

Prepared For:

Federal Emergency Management Agency (FEMA) Region III

Pendleton County, West Virginia

October 2020

Table of Contents

1.0	STUDY DESCRIPTION	4

1.1	County Description	4

1.2	Purpose of Study	4

1.3	Scope of Work	4

1.4	Types of flooding and flooding history	5

1.5	Other studies	5

2.0	HYDRAULIC ANALYSIS	5

2.1	Study limits and Methodology	5

2.2	Assumptions for HEC-RAS Modeling	10

2.3	Topography	10

2.4	survey	11

2.5	Boundary Conditions	11

2.6	Cross Sections	12

2.7	Structures	12

2.8	Coefficients	13

2.9	Ineffective and Storage Areas	13

2.10	Channel Roughness Values	13

2.11	Blocked obstructions	14

2.12	Special Circumstances	14

2.13	Calibration	17

3.0	HYDRAULIC RESULTS	17

3.1	Comparison with effective FIS report	18

3.2	Floodway	31

4.0	FLOODPLAIN MAPPING	31

5.0	REFERENCES	31

6.0	APPENDICES	33

List of Tables

Table 1. Streams Studied Using Approximate Methods	6

Table 2. Streams Studied Using Detail Methods	10

Table 3. Model Starting Conditions for Detail Studied Streams	11

Table 4. Manning’s Roughness Values	13

Table 5. Comparison of BFEs (Effective versus Revised) for Dry Creek 1	18

Table 6. Comparison of BFEs (Effective versus Revised) for Greenbrier River 2	18

Table 7. Comparison of BFEs (Effective versus Revised) for Greenbrier River 3	18

Table 8. Comparison of BFEs (Effective versus Revised) for Greenbrier River 4	19

Table 9. Comparison of BFEs (Effective versus Revised) for Meadow River 3	19

Table 10. Comparison of BFEs (Effective versus Revised) for Wades Creek 1	20

Table 11. Summary of Hydrologic and Hydraulic Analysis	21

List of Figures

Figure 1. Meadow River disaster reach and non-disaster reach	15

Figure 2. Greenbrier River disaster reaches, and non-disaster reaches	16

Figure 3. Greenbrier County Reach Studies Location	35

Appendices

Appendix A - Greenbrier County, WV Studies Map

# STUDY DESCRIPTION

## County Description

Pendleton County is located in the eastern portion of West Virginia. It is bordered by Hardy County, WV to the northeast, Grant County, WV to the north, Randolph County, WV to the west, Pocahontas County, WV to the southwest, Highland County, VA to the south, Augusta County, VA to the southeast, and Rockingham County, VA to the east. Pendleton County is 698 square miles with a county total population of 6,969 (U.S. Census, 2010). Appendix A shows Pendleton County, West Virginia, and the stream reaches included in this study.

## Purpose of Study

The purpose of this study is to conduct a hydraulic study to support the Federal Emergency Management Agency (FEMA) Region III Cooperating Technical Partners Program. The scope of study includes updating and developing approximate (Zone A) studies and select detailed studies (Zone AE). The HUC 8 watersheds covering Pendleton County scoped reaches is 02070001.

## Scope of Work

The scope of work includes the restudy of 401.6 miles of approximate Zone A streams, and 2.6 miles of detailed Zone AE streams. The purpose of this hydraulics report is to document the methodology and calculations used to develop the 10%, 4%, 2%, 1%, 1% plus, and 0.2%-annual-chance exceedance event water surface elevations (WSELs). The location of the approximate study reaches, and their corresponding HUC-8 watershed are illustrated on the map provided in Appendix A.

## Types of flooding and flooding history

All streams in this study are riverine and without tidal influences. According to the 2010 FIS report, the major source of flooding within the study area is the South Branch Potomac River.  A USGS gage is located on the South Branch Potomac River upstream of the Town of Franklin. 2010 FIS report mentions that there are no existing flood control structures that would provide protection during a major flood in Pendleton County (FEMA, 2010).

# HYDRAULIC ANALYSIS

## Study limits and Methodology

A total of 404.13 miles of stream reaches were modeled for the project. During the development of hydraulic modeling, the stream centerlines were updated to follow aerials and/or topographic datasets. The hydraulic models for streams studied under this task were developed with the USACE Hydraulic Engineering Center (HEC) River Analysis System (RAS), version 5.0.7 (HEC, 2019).  All Zone A modeled streams were studied by approximate methods and Zone AE modeled stream were studied by detailed methods. One-dimensional steady state hydraulic models (HEC-RAS) were developed to estimate the 10%, 4%, 2%, 1%, 1% plus, and 0.2%-annual-chance exceedance event WSELs.  Table 1 below lists 121 streams segments which were studied to update effective Zone-A flood hazard boundaries. Table 2 below lists one stream which was studied using detail methods.

Table 1. Streams Studied Using Approximate Methods

| No. | WTR_NM | Mileage |
| --- | --- | --- |
| 1 | BACKRN | 0.94 |
| 2 | BIGRN | 12.22 |
| 3 | BLCKTHRNCK | 6.94 |
| 4 | BLCKTHRNCK_T1 | 0.36 |
| 5 | BLIZARDRN | 1.47 |
| 6 | BOUSESRN | 1.04 |
| 7 | BRIERYGRN | 1.19 |
| 8 | BRIGGSRN | 3.07 |
| 9 | BROADRN | 4.81 |
| 10 | BRUSHFK | 5.38 |
| 11 | BRUSHRN | 6.05 |
| 12 | BRUSHRN1 | 3.19 |
| 13 | BUCKLICKRN | 1.97 |
| 14 | CAMPRN | 2.49 |
| 15 | DEERRN | 7.05 |
| 16 | DETIMERRN | 1.19 |
| 17 | DICERN | 2.51 |
| 18 | DICERN1 | 0.76 |
| 19 | DRYRN | 3.47 |
| 20 | DRYRN_T1 | 0.31 |
| 21 | DRYRN1 | 2.28 |
| 22 | DRYRN2 | 2.20 |
| 23 | EDRYRN | 3.24 |
| 24 | ELKRN | 1.70 |
| 25 | FISHERRN | 0.93 |
| 26 | FLESHERRN | 0.75 |
| 27 | FRIENDSRN | 6.15 |
| 28 | GEORGERN | 1.90 |
| 29 | GRAVLCKRN | 0.90 |
| 30 | HAMMERRN | 1.96 |
| 31 | HAMMERRN1 | 2.73 |
| 32 | HAWESRN | 4.45 |
| 33 | HAWESRN_T1 | 1.08 |
| 34 | HEMLOCKRN | 0.34 |
| 35 | HORSCMPRN | 2.89 |
| 36 | JOHNSONRN | 1.34 |
| 37 | KESSNERRN | 1.89 |
| 38 | KETTLECK | 4.21 |
| 39 | KETTLECK1 | 0.76 |
| 40 | LAURELFK | 2.04 |
| 41 | LAURELRN | 2.10 |
| 42 | LICKRN | 2.66 |
| 43 | LITTLECK | 0.84 |
| 44 | LITTLECMRN | 0.51 |
| 45 | LITTLEFK | 7.03 |
| 46 | LONGRN | 2.37 |
| 47 | LONGRN1 | 2.80 |
| 48 | LONGRN2 | 1.39 |
| 49 | LROUGHRN | 2.01 |
| 50 | LSTONYRN | 1.37 |
| 51 | MCINTSHRN | 0.85 |
| 52 | MILLCK | 2.85 |
| 53 | MILLERRN | 4.05 |
| 54 | MILLRN | 5.92 |
| 55 | MOYERRN | 1.87 |
| 56 | NFLONGRN | 0.30 |
| 57 | NFSBPOTMC_T1 | 2.01 |
| 58 | NFSBPOTMC_T1A | 0.62 |
| 59 | NFSBPOTMC_T2 | 1.10 |
| 60 | NFSBPOTOMC | 33.62 |
| 61 | NMILLCK | 3.98 |
| 62 | PADDYRN | 2.22 |
| 63 | PETERSRN | 3.71 |
| 64 | PIKEGRN | 0.24 |
| 65 | PMILLRN | 0.86 |
| 66 | PROPSTRN | 1.49 |
| 67 | PWDRMILRN | 1.14 |
| 68 | REDLCKRN | 1.49 |
| 69 | REEDSCK | 11.25 |
| 70 | REEDSCK_T1 | 0.39 |
| 71 | ROADRN | 2.94 |
| 72 | ROARINGCK | 3.48 |
| 73 | ROARINGCK1 | 0.82 |
| 74 | ROCKYRN | 0.63 |
| 75 | ROOTRN | 1.41 |
| 76 | ROUGHRN | 5.26 |
| 77 | SAWMILBR | 2.44 |
| 78 | SAWMILLRN | 1.77 |
| 79 | SBPOTOMC | 22.45 |
| 80 | SBPOTOMC_T1 | 0.43 |
| 81 | SBPOTOMC_T2 | 0.54 |
| 82 | SBPOTOMC_T3 | 0.62 |
| 83 | SBPOTOMC_T4 | 0.56 |
| 84 | SBPOTOMC_T5 | 0.71 |
| 85 | SBPOTOMC_T6 | 1.82 |
| 86 | SBPOTOMC2 | 15.56 |
| 87 | SENECACK | 19.03 |
| 88 | SENGRN | 0.33 |
| 89 | SENGRN_T1 | 0.29 |
| 90 | SFSBPOTOMC | 39.01 |
| 91 | SFSBPOTOMC_T1 | 1.92 |
| 92 | SFSBPOTOMC_T2 | 1.72 |
| 93 | SFSBPOTOMC_T3 | 0.50 |
| 94 | SHAFTRN | 1.58 |
| 95 | SHAVERN | 2.76 |
| 96 | SMILLCK | 1.67 |
| 97 | SMITHCK | 8.43 |
| 98 | STAUFLERN | 1.81 |
| 99 | STONYCK | 1.29 |
| 100 | STONYCK_T1 | 0.19 |
| 101 | STONYRN | 3.98 |
| 102 | STONYRN1 | 3.08 |
| 103 | STRADERRN | 1.01 |
| 104 | STRAIGHTFK | 0.61 |
| 105 | STRAIGHTRN | 0.34 |
| 106 | STRINGTRN | 0.28 |
| 107 | STRINGTRN_T1 | 0.21 |
| 108 | SUGARRN | 1.38 |
| 109 | TEETERCRN | 1.77 |
| 110 | TETERRN | 1.36 |
| 111 | THORNCK | 9.20 |
| 112 | THORNCK_T1 | 0.46 |
| 113 | TROUTRN | 5.80 |
| 114 | UPGULFRN | 0.53 |
| 115 | VANCERN | 1.63 |
| 116 | VANCERN_T1 | 0.17 |
| 117 | WHITESRN | 2.08 |
| 118 | WHITHORNCK | 7.84 |
| 119 | WILSONRN | 2.87 |
| 120 | WOLFRN | 0.93 |
| 121 | ZEKERN | 0.92 |

Table 2. Streams Studied Using Detail Methods

| No. | Stream Name | Start Description | End Description | Miles |
| --- | --- | --- | --- | --- |
| 1 | South Branch Potomac River 1 | Approximately 3600 feet downstream of confluence with Propst Run | Approximately 500 feet downstream of confluence with Trout Run | 2.6 |

## Assumptions for HEC-RAS Modeling

To use HEC-RAS for one dimensional steady state hydraulic computations for Pendleton County, some assumptions were made: the flow regime was subcritical; the flow was steady and the flow through and around structures was unobstructed.

## Topography

Topographic data for the floodplain models were developed using 4ft resolution Light Detection and Ranging (LiDAR) funded by FEMA (2016 VA FEMA R3 Southwest Lidar).  Data was collected from March 23 to April 27, 2017 in West Virginia. The bare earth Digital Elevation Model (DEM) tiles with 1-m cell size, from the LiDAR deliverable were mosaicked together and projected into a single DEM in the West Virginia State Plate South (ft) coordinate system. The vertical unit of the DEM was converted to foot (NAVD88).

Source documentation, including project metadata, horizontal accuracy assessment reports, LiDAR collection map and vertical accuracy assessment report have been uploaded to the FEMA’s Mapping Information Platform (MIP). Geospatial files such as the defined project areas, tile indices, and contours have been stored under the FEMA Data Capture supplemental data directory in shapefile format and have been uploaded to the MIP.

## survey

Survey data was collected Spring 2020 by Harned Surveying and Engineering, Inc. This survey data was used to model 4 structures modeled for the detailed reach, South Branch Potomac River.  Collected survey data included elevations along the road and on the immediate upstream and downstream faces of the structure. The survey included elevation data from the channel, channel centerline, edge of water, top of bank, toe of bank, face of abutment, bridge corners, low chord, high points, centerline of the bridge, piers, elevation shots, end of bridge and the road profile.

## Boundary Conditions

For approximate studies, normal depth method was used for the starting water surface elevations for all streams that do not share an extent boundary. The normal depth is defined by the slope of stream channel. For areas that share an extent boundary or that were tying into the restudied detailed streams or another restudied approximate reach, Known Water Surface Elevation (KWSEL) was used for the boundary conditions. Table 3 below lists starting conditions applied for stream studied using detail methods.

Table 3. Model Starting Conditions for Studied Detailed Stream

| Stream Name | Model Name | Study Type | Boundary Condition | Downstream Tie-in Reach |
| --- | --- | --- | --- | --- |
| South Branch Potomac River 1 | SBPOTOMC1 | Detailed | KWSE | approximate reach, South Branch Potomac River SBPOTOMC |

According to FEMA Guidance for Flood Risk Analysis and Mapping, Hydraulics: One-Dimensional Analysis, the assumption of coincident peaks may be appropriate if all the following are true:

- The ratio of the drainage areas lies between 0.6 and 1.4.

- The arrival times of flood peaks are similar for the two combining watersheds.

- The likelihood of both watersheds being covered by the storm being modeled is high.

According to FEMA guidance, for starting conditions on tributaries, the Mapping Partner should use normal depth unless a coincident peak situation is assumed, or the tributary flow depths are higher than the corresponding mainstream events.

Below is the list of streams that have been identified and met the requirements for coincident peaks and hydraulics modeling were completed using known water surface elevation as the boundary condition.

Table 4. Model Starting Conditions for Studied Approximate Stream

| Full Name of Reach | AFG Name | Mileage | Boundary Condition | Downstream Tie-in Reach/coincident Peak with |
| --- | --- | --- | --- | --- |
| Back Run | BACKRN | 0.9 | KWSEL | Sawmill Branch |
| Blackthorn Creek | BLCKTHRNCK | 6.96 | KWSEL | THORNCK |
| Camp Run | CAMPRN | 2.27 | KWSEL | KETTLECK |
| Kessner Run | KESSNERRN | 1.86 | KWSEL | South Mill Creek |
| Paddy Run | PADDYRN | 2.2 | KWSEL | TROUT RUN |
| Stony Creek | STONYCK | 1.28 | KWSEL | NMILLCK |
| Stringtown Run | STRINGTRN | 0.31 | KWSEL | Root Run |
| Whitethorn Creek | WHITHORNCK | 7.75 | KWSEL | THORNCK |

## Cross Sections

Floodplain cross sections were placed perpendicular to flow at representative locations along the modeled streams.  Cross sections were spaced at closer intervals at locations of sudden changes in stream geometry or direction as well as areas of expansion and contraction. At the structures, sections were located to follow the toe of the embankment of the structure, and to capture expansion and contraction of the floodplain, following guidelines from the HEC-RAS Reference Manual (HEC-RAS River Analysis System, 2016). The geometric information for the cross sections was extracted from the, 3.2 ft cell-size raster, LiDAR based topographic information.

For the detailed studies, cross section topographic data included LiDAR data, survey data located at the structures, and invert data from the effective models. Channel bathymetry data was determined from the best available data. Survey data collected in 2020 was considered the best available data for modeling the channel bathymetry.

The cross sections included within the models are depicted spatially (digitally) in the S_XS feature dataset and submitted to MIP within Hydraulics Data Capture in this DCS Hydraulics Data submittal. The hydraulic HEC-RAS models are also included in this submittal.

## Structures

There are 4 structures conveying flood flows along the detailed studied reach. These structures were field surveyed. The details and locations of surveyed structures were documented with a DCS compliant survey upload.

The structures were coded into HEC-RAS geometry based off the field survey data and photographs. LIDAR data was used to extend the top of road profile beyond the limits of the survey information. Survey data was augmented by orthoimagery and LiDAR. The structures were typically modeled using the Energy (Standard Step) method for Low Flow conditions and Pressure and/or Weir method for High Flow conditions, but exceptions were made for structures exhibiting different flow conditions. Additional details specific to input data were also included in the “Description” field in the HEC-RAS geometry files.

## Coefficients

Contraction and expansion coefficients of 0.1 and 0.3 respectively were utilized within the analysis. Contraction and expansion coefficients were increased to 0.3 and 0.5, respectively, at the structures’ upstream and downstream bounding cross sections and at the upstream approach section.

## Ineffective and Storage Areas

Ineffective flow areas were used to model areas that provide storage within a cross section, but for which there is insignificant conveyance.  These areas were set as appropriate near structures and in areas where natural contraction or expansion of the floodplain is apparent.  Ineffective flow areas, modeled in the approximate studies, were applied visually based upon review of the delineated floodplain boundaries, LIDAR based topographic data and aerials.  Ineffective flows were added at basin boundaries for likely split flow locations.

## Channel Roughness Values

The channel and overbank Manning’s n values were based upon the NLCD 2016 Landuse/Landcover (NLCD 2016).  Channel Manning’s n values typically ranged between 0.025-0.055. The values for different land covers are presented below in Table 5. For detailed streams, the information within the Landuse/Landcover dataset was supplemented with aerial imagery to better define roughness coefficients.

Table 5. Manning’s Roughness Values

| Class Code | Type | Manning’s n value |
| --- | --- | --- |
| N/A | Main channel | 0.045 |
| 11 | Open Water | 0.025 |
| 21 | Developed, Open Space | 0.06 |
| 22 | Developed, Low Intensity | 0.055 |
| 23 | Developed, Medium Intensity | 0.035 |
| 24 | Developed, High Intensity | 0.02 |
| 31 | Barren Land (Rock/Sand/Clay) | 0.06 |
| 41 | Deciduous Forest | 0.1 |
| 42 | Evergreen Forest | 0.12 |
| 43 | Mixed Forest | 0.1 |
| 51 | Dwarf Scrub | 0.1 |
| 52 | Shrub/Scrub | 0.1 |
| 71 | Grassland/Herbaceous | 0.055 |
| 81 | Pasture/Hay | 0.05 |
| 82 | Cultivated Crops | 0.05 |
| 90 | Woody Wetlands | 0.045 |
| 95 | Emergent Herbaceous Wetlands | 0.07 |

## Blocked obstructions

For streams studied using detailed methods, blocked obstructions were coded for cross sectional segments overlaying buildings. The building data was obtained from the US Buildings Layer developed by Microsoft and available under the Open Data Commons Open Database License (US Building Footprints).  The elevations for blocked obstructions were defined to be greater than all modeled recurrence events.

## Special Circumstances

Within Pendleton County, there are 19 dams in the National Inventory of Dams that are on tributaries of South Fork South Branch Potomac River. These dams were built in the 1960s and 1970s and are listed as serving the purpose of flood control (USACE NID). All these dams are located on approximate reaches. To model these dams conservatively, centerline was edited to go through the spillway instead of through the structure.

# HYDRAULIC RESULTS

Widening and narrowing of the 1-percent-annual-chance floodplain extent compared to the current effective, was observed throughout the county. Meadow River, dry creek and Wades Creek floodways follow the current regulatory floodway mostly and they narrow or widen slightly at few places. Floodways were not established for Greenbrier rivers previously and they were created during this study.

Most streams experienced both increases and decreases when comparing the computed model WSELs to the current regulatory base flood elevations (BFEs). Changes in water surface elevations could be a result of implementing updated topography and hydrology data.

## Comparison with effective FIS report

The revised base (1-percent-annual-chance) flood elevations (BFEs) were compared to the effective BFEs at letter cross section locations. Table 6 compares the BFEs and statistics from the comparisons. The comparisons revealed both increases as well decrease in BFEs for all studied streams.  Summary of hydrologic and hydraulics analysis for studied streams is also presented in Table 11 below.

Table 6. Comparison of BFEs (Effective versus Revised) for South Branch Potomac River 1

| River Station | Computed Water Surface Elevation (ft) | Current Regulatory Water Surface Elevation (ft) | Difference in Water Surface Elevation (ft) |
| --- | --- | --- | --- |
| 855 | 1657.3 | 1650.6 | 6.7 |
| 3276 | 1663.5 | 1662.3 | 1.2 |
| 5590 | 1671.1 | 1672 | -0.9 |
| 6802 | 1676.3 | 1675.1 | 1.2 |
| 8424 | 1686.1 | 1681.9 | 4.2 |
| 9883 | 1691.9 | 1689.2 | 2.7 |
| 11003 | 1698.6 | 1694.8 | 3.8 |
| 13474 | 1717.5 | 1714.7 | 2.8 |

The increase in water surface elevation may be attributed to the increase in discharges compared to the effective discharges. The effective flows presented in the FIS report dated March 2, 2010 for South Branch Potomac River were computed prior to the 2010 county-wide revisions. The program HECWRC was used to compute discharges following criteria from USGS Bulletin 17B (FEMA, 2010). The only discharge provided in the 2010 Pendleton County FIS was the 1-percent-annual-chance on South Branch Potomac River. Another source used for comparison was the flow provided in the SIR2010-5033 report. This value is based off a 17B analysis using flow values up to the 2006-2007 water year and weighted with the regional regression equation.

For this study, Wood performed an updated 17C gage analysis for Pendleton County. Bulletin 17C is an improved statistical method from Bulletin 17B, and the Wood analysis for South Branch Potomac River included peak flow records up to the 2019 water year. The resulting flow was weighted with 2010 regression and gage-weighting equations (Wiley and Atkins, 2010). The difference between the three flow results may thus be attributed to additional peak flow data, updated regression equations, and updated statistical methods. Further information on hydrologic analysis can be found in the hydrology report.

The spatial data pertaining to the hydraulic analyses was submitted digitally to FEMA’s Mapping Information Platform (MIP) meeting Data Capture Technical Reference, February 2019 version (FEMA, 2019a).  The spatial data is projected to GCS_North_American_1983 and has a horizontal datum of NAD83 according to Flood Insurance Rate Map (FIRM) Database Technical Reference, February 2019 version (FEMA, 2019b).

Table 12. Summary of Hydrologic and Hydraulic Analysis

| Flooding Source | Study Limits Downstream Limit | Study Limits Upstream Limit | Hydrologic Model or Method Used | Hydraulic Model or Method Used | Date Analyses Completed | Flood Zone on FIRM | Special Considerations |
| --- | --- | --- | --- | --- | --- | --- | --- |
| South Branch Potomac River 1 | Approximately 500 feet downstream of confluence with Trout Run | Approximately 3600 feet downstream of confluence with Propst Run | 17C Gage Analysis and Gage-weighted with Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | AE |  |
| BACKRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| BIGRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| BLCKTHRNCK | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| BLCKTHRNCK_T1 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| BLIZARDRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| BOUSESRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| BRIERYGRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| BRIGGSRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| BROADRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| BRUSHFK | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| BRUSHRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| BRUSHRN1 | Downstream Extents | Upstream Extents | Gage Analysis weighted with Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| BUCKLICKRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| CAMPRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| DEERRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| DETIMERRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| DICERN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| DICERN1 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| DRYRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| DRYRN_T1 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| DRYRN1 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| DRYRN2 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| EDRYRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| ELKRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| FISHERRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| FLESHERRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| FRIENDSRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| GEORGERN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| GRAVLCKRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| HAMMERRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| HAMMERRN1 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| HAWESRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| HAWESRN_T1 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| HEMLOCKRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| HORSCMPRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| JOHNSONRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| KESSNERRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| KETTLECK | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| KETTLECK1 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| LAURELFK | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| LAURELRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| LICKRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| LITTLECK | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| LITTLECMRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| LITTLEFK | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| LONGRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| LONGRN1 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| LONGRN2 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| LROUGHRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| LSTONYRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| MCINTSHRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| MILLCK | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| MILLERRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| MILLRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| MOYERRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| NFLONGRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| NFSBPOTMC_T1 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| NFSBPOTMC_T1A | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| NFSBPOTMC_T2 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| NFSBPOTOMC | Downstream Extents | Upstream Extents | Gage Analysis weighted with Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| NMILLCK | Downstream Extents | Upstream Extents | Gage Analysis weighted with Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| PADDYRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| PETERSRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| PIKEGRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| PMILLRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| PROPSTRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| PWDRMILRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| REDLCKRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| REEDSCK | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| REEDSCK_T1 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| ROADRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| ROARINGCK | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| ROARINGCK1 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| ROCKYRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| ROOTRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| ROUGHRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| SAWMILBR | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| SAWMILLRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| SBPOTOMC | Downstream Extents | Upstream Extents | Gage Analysis weighted with Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| SBPOTOMC_T1 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| SBPOTOMC_T2 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| SBPOTOMC_T3 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| SBPOTOMC_T4 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| SBPOTOMC_T5 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| SBPOTOMC_T6 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| SBPOTOMC2 | Downstream Extents | Upstream Extents | Gage Analysis weighted with Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| SENECACK | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| SENGRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| SENGRN_T1 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| SFSBPOTOMC | Downstream Extents | Upstream Extents | Gage Analysis of Regulated Flows | HEC-RAS 5.0.7 | October 2020 | A |  |
| SFSBPOTOMC_T1 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| SFSBPOTOMC_T2 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| SFSBPOTOMC_T3 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| SHAFTRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| SHAVERN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| SMILLCK | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| SMITHCK | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| STAUFLERN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| STONYCK | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| STONYCK_T1 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| STONYRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| STONYRN1 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| STRADERRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| STRAIGHTFK | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| STRAIGHTRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| STRINGTRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| STRINGTRN_T1 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| SUGARRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| TEETERCRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| TETERRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| THORNCK | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| THORNCK_T1 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| TROUTRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| UPGULFRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| VANCERN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| VANCERN_T1 | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| WHITESRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| WHITHORNCK | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| WILSONRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| WOLFRN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |
| ZEKERN | Downstream Extents | Upstream Extents | Regional Regression Equations | HEC-RAS 5.0.7 | October 2020 | A |  |

## Floodway

No floodways were computed for Pendleton County as part of this study.

# FLOODPLAIN MAPPING

The floodplains for all stream reaches were mapped utilizing the aforementioned 4 ft DEM based on 1-percent-annual-chance flood elevations at modeled cross sections. Water surface elevations between model cross sections were interpolated. The water surface and DEM were intersected with each other to create raster inundation areas. The raster is converted to polygons which were then smoothed with a tolerance of 25 feet to remove the boundary jaggedness originated from the raster input

# REFERENCES

Federal Emergency Management Agency (FEMA). 2019. Data Capture Technical Reference, February 2019a.

Federal Emergency Management Agency (FEMA). 2019. Flood Insurance Rate Map (FIRM) Database Technical Reference, February 2019b.

Federal Emergency Management Agency, 2010. Flood Insurance Study, Pendleton County, West Virginia and Incorporated Areas, Effective: March 2,2010

Hydrologic Engineering Center, HEC-RAS River Analysis System, Version 5.0.7, U.S. Army Corps of Engineers, Davis, California, March 2019.

Hydrologic Engineering Center, HEC-RAS River Analysis System Hydraulic Reference Manual Version 5.0, U.S. Army Corps of Engineers, Davis, California, February 2016.

United States Census Bureau, 2010. Quick Facts – Pendleton County, West Virginia, Accessed online October 2020 online at https://www.census.gov/quickfacts/pendletoncountywestvirginia

U.S. Building Footprints, Microsoft Open Source GitHub, downloaded January 2019 https://github.com/Microsoft/USBuildingFootprints https://usbuildingdata.blob.core.windows.net/usbuildings-v1-1/WestVirginia.zip

U.S. Department of the Interior, Geological Survey, National Land Cover Database. 2016. Accessed online July 2020 at: https://www.mrlc.gov/data?f%5B0%5D=category%3Aland%20cover&f%5B1%5D=year%3A2016

U.S. Geological Survey, 2018, USGS Lidar Point Cloud VA FEMA R3 Southwest B 2016 Y133 LAS 2018: U.S. Geological Survey.

# APPENDICES

APPENDIX A

Pendleton County, WV Studies Map

Figure 3. Pendleton County Reach Studies Location