Hydraulic Project Narrative

Pendleton County, West Virginia

Prepared By:

Wood Environment & Infrastructure Solutions, INC.

4795 Meadow Wood Lane, Suite 310E

Chantilly, VA 20151

Federal Emergency Management Agency (FEMA) Region III

Cooperating Technical Partners Program 
Mapping Activity Statement No. WVDHSEM-2016-2

Prepared For:

Federal Emergency Management Agency (FEMA) Region III

Pendleton County, West Virginia

October 2020

Table of Contents

1.0	INTRODUCTION	3

1.1	PROJECT OverviEW	3

1.2	SCOPE OF WORK	3

2.0	ADDITIONAL INFORMATION	3

2.1	ISSUEs..	3

2.2	INFORMATION FOR THE NEXT MAPPING PARTNER	3

3.0	REFERENCES	4

4.0	FIGURES	1

List of Figures

Figure 1 - Pendleton County, WV Studies Map

# INTRODUCTION

## PROJECT OverviEW

Pendleton County is located in the eastern portion of West Virginia. It is bordered by Hardy County, WV to the northeast, Grant County, WV to the north, Randolph County, WV to the west, Pocahontas County, WV to the southwest, Highland County, VA to the south, Augusta County, VA to the southeast, and Rockingham County, VA to the east. Pendleton County is 698 square miles with a county total population of 6,969 (U.S. Census, 2010). Appendix A shows Pendleton County, West Virginia, and the stream reaches included in this study.

## SCOPE OF WORK

The purpose of this study is to conduct a hydraulic study to support the Federal Emergency Management Agency (FEMA) Region III Cooperating Technical Partners Program. The scope of study includes updating and developing approximate (Zone A) studies and select detailed studies (Zone AE). The HUC 8 watersheds covering Pendleton County scoped reaches is 02070001.

The scope of work includes the restudy of 401.6 miles of approximate Zone A streams, and 2.6 miles of detailed Zone AE streams. The purpose of this hydraulics report is to document the methodology and calculations used to develop the 10%, 4%, 2%, 1%, 1% plus, and 0.2%-annual-chance exceedance event water surface elevations (WSELs). The location of the approximate study reaches, and their corresponding HUC-8 watershed are illustrated on the map provided in Appendix A.

# ADDITIONAL INFORMATION

## ISSUEs

No unresolved issues remain. See the DCS Hydraulics Report for methodology and results.

## INFORMATION FOR THE NEXT MAPPING PARTNER

Please refer to the DCS Hydrology submittal and DCS Hydraulics submittal for all information regarding the Pendleton County, West Virginia.

# REFERENCES

Federal Emergency Management Agency, 2010. Flood Insurance Study, Pendleton County, West Virginia and Incorporated Areas, Effective: March 2,2010

United States Census Bureau, 2010. Quick Facts – Pendleton County, West Virginia, Accessed online October 2020 online at https://www.census.gov/quickfacts/pendletoncountywestvirginia

U.S. Army Corps of Engineers, Hydrologic Engineering Center (HEC), 2019. HEC-RAS River Analysis System, Version 5.0.7, U.S. Army Corps of Engineers (USACE).

# FIGURES

Figure 1. Pendleton County Reach Studies Location