Metadata,

Full Building-Level Risk Assessment (BLRA) for Primary Structures

WV GIS Technical Center

West Virginia University

The full Building-Level Risk Assessment (BLRA) table integrates building attributes from two tabular reports: Building Inventory Enhanced (BIE) and Building Input FAST (BIF), as outlined in the following tables.

Note: In this document, data fields are grouped into categories, so their order may differ from that in the BLRA table.

Building Inventory Enhanced (BIE) Attributes or Added at the BLRA Stage (Pages 1 to 9):

| Field Name | Short Description | Category | Data Source | Data Type | Units | Sample Value from BLRA |
| --- | --- | --- | --- | --- | --- | --- |
| Lat | Latitude based on NAD 83 | Spatial Identifiers | WVGISTC: GIS projects | Numeric | N/A | 37.98276 |
| Long | Longitude based on NAD 83 | Spatial Identifiers | WVGISTC: GIS projects | Numeric | N/A | -81.823027 |
| Plus_Code | Google Plus Code (11-Digits) | Spatial Identifiers | Google | Text* | N/A | 869WX5MG+4Q5 |
| Building_ID | Unique building identifier (16-digit GIS parcel identifier_Address No.) | Spatial Identifiers | WV tax assessment records; Local E911 Addressing Offices; WVGISTC | Text* | N/A | 03-08-0026-0082-0000_15908 |
| Building_Type | Primary building (P code) or Unknown (U code) | Spatial Identifiers | WVGISTC | Text* | N/A | P |
| Full_E-911_Address | Complete E-911 address | Spatial Identifiers | Local E911 Addressing Offices | Text | N/A | 15908 SPRUCE RIVER RD, JEFFREY, WV, 25114 |
| GIS_Parcel_ID | 16-digit GIS parcel identifier | Spatial Identifiers | WV tax assessment records | Text* | N/A | 03-08-0026-0082-0000 |
| IAS_ID | Assessment record override Parcel ID (blank if no override) | Spatial Identifiers | WV tax assessment records | Text* | N/A | - |
| WV_Flood_Tool_Link | WV Flood Tool link (Risk MAP view) | Spatial Identifiers | WVGISTC | Text | N/A | https://mapwv.gov/flood/map/?wkid=102100&x=-9108497.752679156&y=4576990.723588105&l=13&v=2 |
| WV_Parcel_Assessment_Link | WV detailed tax assessment report link | Spatial Identifiers | WV tax assessment records | Text | N/A | https://mapwv.gov/Assessment/Detail/?PID=03080026008200000000 |

* Numbers processed as text

| Field Name | Short Description | Category | Data Source | Data Type | Units | Sample Value from BLRA |
| --- | --- | --- | --- | --- | --- | --- |
| CID | FEMA 6-digit community identifier | Geographic Units | FEMA | Text* | N/A | 540007 |
| Community_Name | Community in which the structure is located | Geographic Units | - | Text | N/A | Boone County* - Unincorporated |
| COMMUNITY | Community in which the structure is located | Geographic Units | - | Text | N/A | Boone County* - Unincorporated |
| County | Name of the county in which the structure is located | Geographic Units | - | Text | N/A | Boone |
| Incorporated_Unincorporated | Incorporated place or unincorporated area | Geographic Units | - | Text | N/A | Unincorporated |
| WVRPDC_Region | Number of the WV Regional Planning and Development Council region in which the structure is located | Geographic Units | - | Text* | N/A | PDC Region 3 |
| Stream_Name | Stream name of which the building is located in the floodplain | Geographic Units | U.S. Geological Survey, National Hydrography Dataset | Text | N/A | Spruce Fork |
| Watershed_HUC8 | Watershed name (HUC-8) in which the building is located | Geographic Units | Natural Resources Conservation Service | Text | N/A | Coal |
| HUC8_Code | Watershed code (HUC-8) in which the building is located | Geographic Units | Natural Resources Conservation Service | Text* | N/A | 5050009 |
| Flood_Zone_Designation | Flood zone designation (Flood hazard area type: A, AE, Advisory A, Updated AE, X, etc.) | Flood Zone | FEMA Flood Insurance Rate Map (FIRM) | Text | N/A | AE |
| Floodway | If the structure is in Floodway (Yes or No) | Flood Zone | FEMA Flood Insurance Rate Map (FIRM) | Text | N/A | Yes |
| Non_Regulatory | Regulatory zones or Non-Regulatory High Risk Advisory zones | Flood Zone | FEMA Flood Insurance Rate Map (FIRM) | Text | N/A | Regulatory |
| Symbol_FloodRiskZone | Future SFHA change status (Mapped-In, Mapped-In Floodway, Mapped-out, Remain Same, Remain Same Floodway, SFHA New Floodway-followed by the general occupancy: Residential, Commercial, or Other) | Flood Zone | FEMA Flood Insurance Rate Map (FIRM); Non-regulatory flood Zones; WVGISTC: Building-level Inventory | Text | N/A | Remains Same Floodway-Other |

* Numbers processed as text

| Field Name | Short Description | Category | Data Source | Data Type | Units | Sample Value from BLRA |
| --- | --- | --- | --- | --- | --- | --- |
| FloodPlainType_RiskLayer | Floodplain risk layer symbol used in Riskmap view of the Flood Tool including all floodplain types intersecting with the building and the Flood_Depth_Source in parentheses | Flood Zone | FEMA Flood Insurance Rate Map (FIRM); WVGISTC | Text | N/A | Effective AE-Floodway + Updated AE (HEC-RAS) |
| SFHA_TF | If the structure is in the effective Special Flood Hazard Area (SFHA) (T: True) or in the other floodplains (F: False) | Flood Zone | FEMA Flood Insurance Rate Map (FIRM); Non-regulatory flood Zones; WVGISTC: Building-level Inventory | Text | N/A | T |
| Land_Use_Code | Land use code from tax assessment (100 to 700) | Building Characteristics | WV tax assessment records | Numeric | N/A | 612 |
| Land_Use_Description | Land use description from tax assessment | Building Characteristics | WV tax assessment records | Text | N/A | School |
| Hazard_Occupancy_Code | Specific Hazus occupancy class code (RES1 to RES6, COM1 to COM10, IND1 to IND6, AGR1, REL1, GOV1, GOV2, EDU1, or EDU2) | Building Characteristics | WV tax assessment records: Land use codes; User-defined values | Text | N/A | EDU1 |
| General_Occupancy_Code | Generalized Hazus occupancy class including seven classes of Agriculture, Commercial, Education, Government, Industrial, Religious, or Residential | Building Characteristics | WV tax assessment records: Land use codes; User-defined values | Text | N/A | Education |
| GENERAL_OCCUPANCY | Generalized Hazus occupancy class including two classes (Residential: Including 1 to 4 units, or Non-Residential: Other than above) | Building Characteristics | WV tax assessment records: Land use codes; User-defined values | Text | N/A | Non-Residential |
| Owner_Name_s | Owner(s) name(s) | Building Characteristics | WV tax assessment records | Text | N/A | BOARD OF EDUCATION BOONE CO RAMAGE GRADE |
| Full_Owner_Address | Owner(s) address | Building Characteristics | WV tax assessment records | Text | N/A | MADISON, WV 25130 |
| Property_Class_Code | Property class codes from tax assessment (R , F, C, I, A, U, or X for Residential, Farm, Commercial, Industrial, Apartment, Utility, or Exempt respectively) | Building Characteristics | WV tax assessment records | Text | N/A | X |
| Property_Class_Description | Property class descriptions from tax assessment (Residential, Farm, Commercial, Industrial, Apartment, Utility, or Exempt) | Building Characteristics | WV tax assessment records | Text | N/A | Exempt |

| Field Name | Short Description | Category | Data Source | Data Type | Units | Sample Value from BLRA |
| --- | --- | --- | --- | --- | --- | --- |
| Tax_Class | Tax class including 2: Owner-occupied residential and farm land used for agricultural purposes, 3: Not owner-occupied situated outside a municipality, or 4: Not owner-occupied inside a municipality (No property currently taxed in Class 1: Property employed exclusively in agriculture) | Building Characteristics | WV tax assessment records | Text* | N/A | 3 |
| Stories | Number of stories (modified in some cases) | Building Characteristics | WV tax assessment records | Numeric | # | 1 |
| Exterial_Wall_Type | Exterior wall type (construction material) from the assessment data | Building Characteristics | WV tax assessment records | Text | N/A | Brick or Stone |
| Architectural_Style | Architectural style of the building from the assessment data | Building Characteristics | WV tax assessment records | Text | N/A | Conventional |
| Year_Built | Building year (A specific year or Unknown) | Building Characteristics | WV tax assessment records | Numeric | N/A | 1951 |
| FIRM_Status | FIRM status based on the construction year: Pre-FIRM; Pre-FIRM (Map Change) which indicates Post-FIRM construction regulated to Pre-FIRM (Mapped into SFHA); Post-FIRM; or Unknown | Building Characteristics | Bldg. Year based on WV tax assessment records; FEMA FIRM effective dates; Older aerial imagery | Text | N/A | Pre-FIRM |
| Grade | Construction grade based on its physical condition (A+, A, A- : Good; B+, B, B- : Average; C+, C, C- : Fair; D+, D, D- : Poor; E+, E: Very Poor; E- : Very Poor/ Unsound; X+, X, or X- : Excellent; S+, S, S- : Superior)** | Building Characteristics | WV tax assessment records | Text | N/A | - |
| Structure_Area | Structure area, in square feet (modified in some cases, where no record was available) | Building Characteristics | WV tax assessment records; GIS measurements; Default areas for mobile homes (if not recorded) | Numeric | Sq Ft | 26,130 |
| Building_Appraisal | Building appraisal value (modified in some cases), estimation based on neighboring buildings' assessments, county-wide estimations for some mobile homes, estimation based on area (RS Means), insurance (BRIM) assessments, or other sources | Building Characteristics | WV tax assessment records, BRIM Insurance, RS Means, Neighborhood Values, or any other available sources | Numeric | $ | 1,224,740 |

** Quality of construction materials graded by appraisers may be subjective and should only be used as a guide.                                      * Numbers processed as text

| Field Name | Short Description | Category | Data Source | Data Type | Units | Sample Value from BLRA |
| --- | --- | --- | --- | --- | --- | --- |
| Building_Value_Source | Building value source: Assessment (IAS), Assessment (IAS) Modified, Assessment (IAS) Neighbor, Single wide Mobile Home, Double wide Mobile Home, Area (RS Means), Insurance (BRIM), or Other | Building Characteristics | See Bldg. Appraisal. | Text | N/A | Assessment (IAS) Modified |
| Symbol_Bldg_Exposure_Cost_USD | Class of the building exposure cost in US Dollar based on the Flood Tool symbology (>$1M; $250,000-$1M; $100,000-$250,000; $50,000-$100,000; or 0-$50,000) | Building Characteristics | See Bldg. Appraisal. | Text | $ | >1M |
| Dwelling_Value | Dwelling value of the property from the assessment data | Building Characteristics | WV tax assessment records | Numeric | $ | 0 |
| Commercial_Value | Commercial value of the property from the assessment data | Building Characteristics | WV tax assessment records | Numeric | $ | 0 |
| OBY_Value | Other building/yard values of the property from the assessment data | Building Characteristics | WV tax assessment records | Numeric | $ | 550,310 |
| Total_Structures | Number of buildings (Cards) in the property from the assessment data | Building Characteristics | WV tax assessment records | Numeric | # | 1 |
| Accessory_Structures_Count | Number of items (Lines) listed under Other Building and Yard Improvements in the assessment data | Building Characteristics | WV tax assessment records | Numeric | # | 8 |
| Units | Number of units in the property as mentioned in the assessment data | Building Characteristics | WV tax assessment records | Numeric | # | 0 |
| Basement_Type | Basement type from the assessment data including: Crawl, First Basement, Full, Part, Sub Basement, None, or blank if not mentioned in the tax data | Building Characteristics | WV tax assessment records | Text | N/A | First Basement |
| Foundation_Type | Foundation type - Hazus LUT (modified in certain cases) including: Basement, Crawlspace, Fill, Pier, Pile, Slab-on-Grade, or Solid Wall) | Building Characteristics | WV tax assessment records; Elevation Certificates; Building pictures; Field verification | Text | N/A | Slab-on-Grade |
| First_Floor_Height | First floor height based on Foundation Type, Elevation Certificates, or other verifications | Building Characteristics | Foundation based on WV tax assessment records; Elevation Certificates; Building pictures; Field verification | Numeric | Ft | 1.0 |

| Field Name | Short Description | Category | Data Source | Data Type | Units | Sample Value from BLRA |
| --- | --- | --- | --- | --- | --- | --- |
| Flood_Depth_Value | Flood depth grid value, in feet (N/A if no flood depth estimated or Flood_Depth_Source = N/A) | Building Damage Estimate | FEMA RiskMAP Restudies; Non-Restudies; Hazus (not model supported) | Numeric | Ft | 5.5 |
| Flood_Depth_Source | Flood depth source (HEC-RAS, HAZUS, Modified, or N/A) | Building Damage Estimate | See Flood Depth. | Text | N/A | HEC-RAS |
| WSEL_Value | Water Surface Elevation, in feet | Building Damage Estimate | See Flood Depth. | Numeric | Ft | 764.5 |
| WSEL_Source | Water Surface Elevation source: AFH (Advisory Flood Heights), BFE (Effective), UAE (Updated AE) or N/A if none | Building Damage Estimate | See Flood Depth. | Text | N/A | UAE |
| Ground_Elevation | Ground Elevation, in feet | Building Damage Estimate | Statewide Digital Elevation Model (DEM) | Numeric | Ft | 758.8 |

| Field Name | Short Description | Category | Data Source | Data Type | Units | Sample Value from BLRA |
| --- | --- | --- | --- | --- | --- | --- |
| Critical_Infrasturcture | Essential Facilities type (911 Center, Fire Station, Hospital, Nursing home, Police Station, or School) or Non-Historical Community Assets type (College / University, EMS, Government, Religious Institutions, Utility, or Other) or None if not any of these significant structures | Significant Structures | Emergency Management Division; Department of Education; USA Reference; Department of Transportation; Homeland Infrastructure Foundation-Level DATA; WV Water Development Authority; WV Infrastructure Jobs Development Council; WV Division of Natural Resources; WV tax assessment records; WVGISTC; Community feedback | Text | N/A | School |
| Governmental_Building | Government building type (F for federal, S for state, L for local, or blank if none) | Significant Structures | Owner(s) based on WV tax assessment records | Text | N/A | L |
| Historical_Structure | If the structure is a Historical Community Asset (Yes) or not (blank) | Significant Structures | National Register site and area designations; WV tax assessment records; WVGISTC; Community feedback | Text | N/A | - |
| Federal_Land | If the building is on a federally-owned land, by United States of America, (FED) or not (blank) | Significant Structures | Owner(s) based on WV tax assessment records | Text | N/A | - |

| Field Name | Short Description | Category | Data Source | Data Type | Units | Sample Value from BLRA |
| --- | --- | --- | --- | --- | --- | --- |
| Average_Household_Size | Average number of individuals per household in the incorporated place or unincorporated area, where the building is located | People | Census Bureau: 2021 American Community Survey (ACS) 5-year estimates | Numeric | # | 2.5 |
| Residential_Units_FLD | Estimated number of residential units located at the estimated flood depths of >= 1 ft. (Considered maximum number of units in property based on the Hazard_Occupancy_Code including RES1&2: 1 unit, RES3A: 2 units, RES3B: 4 units, RES3C: 9 units, RES3D: 19 units, RES3E: 49 units, RES3F: 75 units, COM1 if Land_Use_Description=Mixed Commercial/Residential: 1 units, other: 0 units). | People | WV tax assessment records: Land use codes; User-defined values; WVGISTC | Numeric | # | 0 |
| Displaced_Population_FLD_BLD | Estimated number of displaced (evacuated) people from the property, for short-term, by inundation of 1 ft or deeper by a 100-year flood calculated as (Residential_Units_FLD x Average_Household_Size) | People | WVGISTC | Numeric | # | 0 |

| Field Name | Short Description | Category | Data Source | Data Type | Units | Sample Value from BLRA |
| --- | --- | --- | --- | --- | --- | --- |
| Comments | General comments about the building or the identification process | Other | WVGISTC | Text | N/A | Ramage Elementary School, Couldn't be found in the BRIM data, The town name and number verified. |
| Data_Issue_1 | Data issue flag 1 including (AD) Address: Incomplete, (AD) Address: None or missing, (AD) Address: None or missing (retrieved from Google Maps), (AD) Address: Wrong E-911 address, (IM) Imagery: Building not visible, (PA) Assmt: Different Assessment Record, (PA) Assmt: No Assessment Record, (PG) Parcel: Geometry Misalignment, (PG) Parcel: Site Address Outside Parcel, (PG) Parcel: No GIS Parcel Geometry, or blank if no issues | Other | WVGISTC | Text | N/A | (AD) Address: None or missing |
| Data_Issue_2 | Data issue flag 2, see Data_Issue_1. | Other | WVGISTC | Text | N/A | (PA) Assmt: Different Assessment Record |
| Timestamp | Time stamp indicating the date when the last Building Inventory Enhanced (BIE) script was run for the county (where the building is located) | Other | WVGISTC | Date | N/A | 6/9/2023 |
| COMMUNITY COMMENTS | Notes about the specific structure based on community feedback or observations (such as flood depth issues, comments about elevated buildings, etc.) | Other | Field verification; Community feedback; Building pictures; etc. | Text | N/A | - |
| EC | If there is an elevation certificate for the building (Y: Yes or N: No) | Other | WVGISTC | Text | N/A | N |
| MS | If the structure is mitigated (Y: Yes or N: No) | Other | WVGISTC | Text | N/A | N |

Building Input FAST (BIF) Attributes (Pages 10 to 17):

| Field Name | Short Description | Category | Data Source | Data Type | Units | Sample Value from BLRA |
| --- | --- | --- | --- | --- | --- | --- |
| Latitude | Same as Lat: Latitude based on NAD 83 | Spatial Identifiers | WVGISTC: GIS projects | Numeric | N/A | 37.98276 |
| Longitude | Same as Long: Longitude based on NAD 83 | Spatial Identifiers | WVGISTC: GIS projects | Numeric | N/A | -81.823 |
| Building_ID | Unique building identifier (16-digit GIS parcel identifier_Address No.) | Spatial Identifiers | WV tax assessment records; Local E911 Addressing Offices; WVGISTC | Text* | N/A | 03-08-0026-0082-0000_15908 |
| Full_E-911_Address | Complete E-911 address | Spatial Identifiers | Local E911 Addressing Offices | Text | N/A | 15908 SPRUCE RIVER RD, JEFFREY, WV, 25114 |
| GIS_Parcel_ID | 16-digit GIS parcel identifier | Spatial Identifiers | WV tax assessment records | Text* | N/A | 03-08-0026-0082-0000 |
| Plus_Code | Google Plus Code (11-Digits) | Spatial Identifiers | Google | Text* | N/A | 869WX5MG+4Q5 |
| WV_Flood_Tool_Link | WV Flood Tool link (Risk MAP view) | Spatial Identifiers | WVGISTC | Text | N/A | https://mapwv.gov/flood/map/?wkid=102100&x=-9108497.752679156&y=4576990.723588105&l=13&v=2 |
| WV_Parcel_Assessment_Link | WV detailed tax assessment report link | Spatial Identifiers | WV tax assessment records | Text | N/A | https://mapwv.gov/Assessment/Detail/?PID=03080026008200000000 |

* Numbers processed as text

| Field Name | Short Description | Category | Data Source | Data Type | Units | Sample Value from BLRA |
| --- | --- | --- | --- | --- | --- | --- |
| Full_Owner_Address | Owner(s) address | Building Characteristics | WV tax assessment records | Text | N/A | MADISON, WV 25130 |
| Year_Built | Building year (A specific year or Unknown) | Building Characteristics | WV tax assessment records | Numeric | N/A | 1951 |
| FIRM_Status | FIRM status based on the construction year: Pre-FIRM, Post-FIRM, Post-FIRM construction regulated to Pre-FIRM (Mapped into SFHA), or Unknown | Building Characteristics | Bldg. Year based on WV tax assessment records; FEMA FIRM effective dates; Older aerial imagery | Text | N/A | Pre-FIRM |
| Occ | Same as Hazard_Occupancy_Code (used in Hazus damage estimation): Specific Hazus occupancy class code (RES1 to RES6, COM1 to COM10, IND1 to IND6, AGR1, REL1, GOV1, GOV2, EDU1, or EDU2) | Building Characteristics | WV tax assessment records: Land use codes; User-defined values | Text | N/A | EDU1 |
| Cost | Same as Building_Appraisal (used in Hazus damage estimation): Building appraisal value (modified in some cases), estimation based on neighboring buildings' assessments, county-wide estimations for some mobile homes, estimation based on area (RS Means), insurance (BRIM) assessments, or other sources | Building Characteristics | WV tax assessment records, BRIM Insurance, RS Means, Neighborhood Values, or any other available sources | Numeric | $ | 1,224,740 |
| NumStories | Same as Stories (used in Hazus damage estimation): Number of stories (modified in some cases) | Building Characteristics | WV tax assessment records | Numeric | # | 1 |
| FoundationType | Foundation type code (used in Hazus damage estimation) - Hazus LUT (modified in certain cases) including: 1 for Pile, 2 for Pier, 3 for Solid Wall, 4 for Basement, 5 for Crawlspace, 6 for Fill, or 7 for Slab-on-Grade) | Building Characteristics | WV tax assessment records; Elevation Certificates; Building pictures; Field verification | Text* | N/A | 7 |
| FirstFloorHt | Same as First_Floor_Height (used in Hazus damage estimation): First floor height based on Foundation Type, Elevation Certificates, or other verifications | Building Characteristics | Foundation based on WV tax assessment records; Elevation Certificates; Building pictures; Field verification | Numeric | Ft | 1.0 |

* Numbers processed as text

| Field Name | Short Description | Category | Data Source | Data Type | Units | Sample Value from BLRA |
| --- | --- | --- | --- | --- | --- | --- |
| Area | Same as Structure_Area (used in Hazus damage estimation): Structure area, in square feet (modified in some cases, where no record was available) | Building Characteristics | WV tax assessment records; GIS measurements; Default areas for mobile homes (if not recorded) | Numeric | Sq Ft | 26,130 |
| UserDefinedFltyId | Unique FAST model identifier | Building Damage Estimate | Hazus Level 2 Analysis | Text* | N/A | 1725 |
| Depth_Grid | Same as Flood Depth but not rounded to one decimal digit (used in Hazus damage estimation): Flood depth grid value, in feet; N/A values replaced with 0 | Building Damage Estimate | FEMA RiskMAP Restudies; Non-Restudies; Hazus (not model supported) | Numeric | Ft | 5.463745 |
| Depth_in_Struc | Flood depth-in-Structure adjusted flood depth grid at the UDF point, in feet. Simple calculation: If the Depth_Grid is a NoData or -9999 value, value is -9999. Else value is Depth_Grid − First_Floor_Height; Positive values: BFE > FFH, Negative Values: BFE < FFH | Building Damage Estimate | Calculated based on Flood Depth and 1st Floor Height | Numeric | Ft | 4.463745 |
| flExp | If a User Defined Facility (UDF) is exposed to a flood. Simply 0 or 1 by Hazus/FAST script. If the UDF is in a flood depth grid, then the value is 1, regardless of depth-in-structure. | Building Damage Estimate | Hazus Level 2 Analysis; Enhanced Building Inventory (or User-Defined Facility) | Text* | N/A | 1 |
| SOID | Specific occupancy ID. The Hazus-MH Flood shorthand that compresses OccupancyClass, NumStories, and FoundationType into a concise 4- to 5-character code, e.g. R11N for a RES1, no basement, single story. Used to access the look-up tables where the user does not specify a DDF. XXXX for buildings not in the flood zone. | Building Damage Estimate | Hazus Level 2 Analysis; Enhanced Building Inventory (or User-Defined Facility) | Text | N/A | E1LN |

* Numbers processed as text

| Field Name | Short Description | Category | Data Source | Data Type | Units | Sample Value from BLRA |
| --- | --- | --- | --- | --- | --- | --- |
| BDDF_ID | Building Depth Damage Function (DDF): If not provided by the user, defaults will be assigned based on Hazus methodology by computing Specific Occupancy ID based on Occupancy Type, Foundation Type, num stories and flood type. If populated by user, the script will check to ensure that only valid DDFs are used. | Building Damage Estimate | Hazus Level 2 Analysis; Enhanced Building Inventory (or User-Defined Facility); Depth Grids | Text* | N/A | 643 |
| BldgDmgPct | Ratio of the estimated building damage to appraised value. Interpolated from the lookup tables, depending on flood depth/damage curves. Value ranges between 0 and 100. For buildings outside the flood zone, the value is set to 0.0. | Building Damage Estimate | Hazus Level 2 Analysis; Enhanced Building Inventory (or User-Defined Facility); Depth Grids | Numeric | % | 9.463745 |
| BldgLossUSD | Estimated building loss, in US dollars, estimated for a 1%-annual-chance flood. Formula: Cost × BldgDmgPct | Building Damage Estimate | Hazus Level 2 Analysis; Enhanced Building Inventory (or User-Defined Facility); Depth Grids | Numeric | $ | 115,906 |
| Symbol_BldgLoss_USD | Class of the building damage estimate (100-yr flood) in US Dollar based on the Flood Tool symbology (>$100K; $50-$100K; $1K-$50K; or < $1K or No Depth Value) | Building Damage Estimate | See BldgLossUSD. | Text | $ | >100K |
| Symbol_Bldg_Damage_Percent | Class of the building damage estimate percentage to the appraised value based on the Flood Tool symbology including 50-100% (Substantial Damage); 10-50% (Moderate Damage); 1-10% (Slight Damage); or <1% or No Depth Value | Building Damage Estimate | See BldgDmgPct. | Text | $ | 1-10% (Slight Damage) |

* Numbers processed as text

| Field Name | Short Description | Category | Data Source | Data Type | Units | Sample Value from BLRA |
| --- | --- | --- | --- | --- | --- | --- |
| ContentCostUSD | Estimated content cost: Building content cost, in US dollars. If user supplied a ContentCost attribute, and the record’s value is non-null, the value is ContentCost. Otherwise, depending on OccupancyClass, it is calculated at 0.5, 1.0, or 1.5 times the user- supplied building Cost. See Hazus-MH Flood Technical Manual (FEMA, 2011). | Building Damage Estimate | Hazus Level 2 Analysis; Enhanced Building Inventory (or User-Defined Facility); Depth Grids | Numeric | $ | 1,224,740 |
| CDDF_ID | Content Depth Damage Function ID. If not provided by the user, defaults will be assigned based on Hazus methodology by computing Specific Occupancy ID based on Occupancy Type, Foundation Type, num stories and flood type. If populated by user, the script will check to ensure that only valid DDFs are used. | Building Damage Estimate | Hazus Level 2 Analysis; Enhanced Building Inventory (or User-Defined Facility); Depth Grids | Text* | N/A | 480 |
| ContDmgPct | Estimated building content damage percentage. Interpolated from the lookup tables, depending on flood depth. Value ranges between 0 and 100. For UDFs outside the flood zone, the value is set to 0.0. | Building Damage Estimate | Hazus Level 2 Analysis; Enhanced Building Inventory (or User-Defined Facility); Depth Grids | Numeric | % | 65.85498 |
| ContentLossUSD | Estimated loss, in US dollars, to the Bbuilding content. Formula: ContDmgPct × ContentCostUSD | Building Damage Estimate | Hazus Level 2 Analysis; Enhanced Building Inventory (or User-Defined Facility); Depth Grids | Numeric | $ | 806552.287793 |
| InventoryCostUSD | Hazus estimates are provided based on Occupancy Class and Area unless provided by the user. Must be greater than or equal to 0. | Building Damage Estimate | Hazus Level 2 Analysis; Enhanced Building Inventory (or User-Defined Facility); Depth Grids | Numeric | $ | 0 |

* Numbers processed as text

| Field Name | Short Description | Category | Data Source | Data Type | Units | Sample Value from BLRA |
| --- | --- | --- | --- | --- | --- | --- |
| IDDF_ID | Inventory Depth Damage Function ID. If not provided by the user, defaults will be assigned based on Hazus methodology by computing Specific Occupancy ID based on Occupancy Type, Foundation Type, num stories and flood type. If populated by user, the script will check to ensure that only valid DDFs are used. | Building Damage Estimate | Hazus Level 2 Analysis; Enhanced Building Inventory (or User-Defined Facility); Depth Grids | Text* | N/A | 0 |
| InvDmgPct | Building Inventory estimated damage percentage. Interpolated from the lookup tables, depending on flood depth. Value ranges between 0 and 100. For UDFs outside the flood zone, the value is set to 0.0. Note that only certain types of OccupancyClass have a standard Inventory Loss function defined. | Building Damage Estimate | Hazus Level 2 Analysis; Enhanced Building Inventory (or User-Defined Facility); Depth Grids | Numeric | % | 0 |
| InventoryLossUSD | Loss, in US dollars, to the inventory content. If user supplied an inventory cost attribute, the value is InvDmgPct × InventoryCostUSD. (Note the significant discrepancy between the computed values and Hazus 4.0 loss estimates. Hazus 4.0 does not correctly implement the Inventory Loss calculation at the UDF level.) | Building Damage Estimate | Hazus Level 2 Analysis; Enhanced Building Inventory (or User-Defined Facility); Depth Grids | Numeric | $ | 0 |
| DebrisID | Debris Function ID | Building Damage Estimate | Hazus Level 2 Analysis; Enhanced Building Inventory (or User-Defined Facility); Depth Grids | Text | N/A | EDU1NBSG4 |
| Debris_Fin | Estimated debris from the finishing of the building | Building Damage Estimate | Hazus Level 2 Analysis; Enhanced Building Inventory (or User-Defined Facility); Depth Grids | Numeric | Tons | 52.26 |

* Numbers processed as text

| Field Name | Short Description | Category | Data Source | Data Type | Units | Sample Value from BLRA |
| --- | --- | --- | --- | --- | --- | --- |
| Debris_Struc | Estimated debris from the structural elements of the building | Building Damage Estimate | Hazus Level 2 Analysis; Enhanced Building Inventory (or User-Defined Facility); Depth Grids | Numeric | Tons | 0 |
| Debris_Found | Estimated debris from the foundation of the building | Building Damage Estimate | Hazus Level 2 Analysis; Enhanced Building Inventory (or User-Defined Facility); Depth Grids | Numeric | Tons | 0 |
| Debris_Tot | Total estimated debris, in tons, in case of a 1%-annual-chance flood. Combines Finish, Structure, and Foundation debris estimates. Based on Occupancy Class, Square Footage, Foundation Type, and Depth-in-Structure. | Building Damage Estimate | Hazus Level 2 Analysis; Enhanced Building Inventory (or User-Defined Facility); Depth Grids | Numeric | Tons | 52.26 |
| Restor_Days_Min | Estimated restoration time, in days— Minimum bound in case of a 1%-annual-chance flood. Note there is no direct Hazus equivalent. The name is identical to what is in the Hazus lookup table. Note that the restoration times assume, like the debris, that a ‘substantially damaged’ structure (one which experiences > 50% loss) is torn down and replaced. | Building Damage Estimate | Hazus Level 2 Analysis; Enhanced Building Inventory (or User-Defined Facility); Depth Grids | Numeric | Days | 450 |
| Restor_Days_Max | Restoration time, in days— Maximum bound in case of a 1%-annual-chance flood. Note there is no direct Hazus equivalent. The name is identical to what is in the Hazus lookup table. Note that the restoration times assume, like the debris, that a ‘substantially damaged’ structure (one which experiences > 50% loss) is torn down and replaced. | Building Damage Estimate | Hazus Level 2 Analysis; Enhanced Building Inventory (or User-Defined Facility); Depth Grids | Numeric | Days | 630 |

| Field Name | Short Description | Category | Data Source | Data Type | Units | Sample Value from BLRA |
| --- | --- | --- | --- | --- | --- | --- |
| GridName | Name of flood depth grid. | Building Damage Estimate | FEMA RiskMAP Restudies; Non-Restudies; Hazus (not model supported) | Text | N/A | Boone_HEC_RAS.tif |